IP assignment

    - Use the software IP Installer to assign the IP address of the IP Camera. The software is in the software CD attached to the product’s package.

    - IP installer supports two languages:

      - IPInstallerCht.exe: Chinese version

      - IPInstallerEng.exe: English version

    - There are 3 kinds of IP configuration.

      - Fixed IP (Public IP or Virtual IP)

      - DHCP (Dynamic IP)

      - Dial-up (PPPoE)

    - Execute IP Installer

    - For Windows XP SP2 users, it may popup the following message box. Click Unblock.

    - IP Installer configuration:

    - IP Installer will search for all the IP Cameras connected on the LAN. The user can click Search Device to search again.

    - Click one of the IP Cameras listed on the left side. The network configuration of this IP camera will be shown on the right side. You can change the name of the IP Camera to your preference (e.g.: Office, warehouse). Change the parameters and click Submit, then click OK, it will apply the changes and reboot the device.

    - Please make sure the subnet of the PC IP address and the IP Camera IP address are the same.

The same Subnet:

IP Camera IP address: 192.168.1.200

PC IP address: 192.168.1.100

Different Subnets:

IP Camera IP address: 192.168.2.200

PC IP address: 192.168.1.100

To Change the PC IP address:

Control PanelNetwork ConnectionsLocal Area Connection PropertiesInternet Protocol (TCP/IP) Properties 
Make sure your IP Camera and PC are int the same Subnet. If not, change the IP Camera subnet or the PC IP subnet accordingly.

    - A quick way to access remote monitoring is to left-click the mouse twice, on a selected IP Camera, listed on Device list of IP Installer. An IE browser will be opened.

    - Then, key-in the default user name: admin and password: admin.