# Milesight Network Video Recorder Application Programming Interface.

# CGI Specification

2018-10-23 V 1.0.7

Milesight Technology Co.,Ltd.

脉视数字技术有限公司

david@milesight.cn

## Table of Contain

Table of Contain	2

Revision history	8

1.1 NETWORK	12

1.1.1 get.network.general	12

1.1.2 set.network.general	13

1.1.3 get.network.port	13

1.1.4 set.network.port	13

1.1.5 get.network.email	14

1.1.6 set.network.email	14

1.1.7 test.network.email	14

1.1.8 get.network.ddns	14

1.1.9 set.network.ddns	15

1.1.10 get.network.pppoe	15

1.1.11 set.network.pppoe	15

1.1.12 get.network.p2p	15

1.1.13 set.network.p2p	16

1.2 SYSTEM	16

1.2.1 get.system.time	16

1.2.2 set.system.time	16

1.2.3 get.system.sysstatus	16

1.2.4 get.system.sysinfo	16

1.2.5 set.system.reboot	17

1.2.6 set.system.devid	17

1.2.7 set.system.shutdown	17

1.3 USER	18

1.3.1 get.user.member	18

1.3.2 get.user.auth	18

1.3.3 get.user.count	18

1.3.4 set.user.member	19

1.3.5 add.user.member	19

1.3.6 del.user.member	19

1.4 EVENT	19

1.4.1 get.event.alarmin	19

1.4.2 set.event.alarmin	20

1.4.3 get.event.alarmout	20

1.4.4 set.event.alarmout	20

1.4.5 get.event.holidays	20

1.4.6 set.event.holidays	21

1.4.7 get.event.videoloss	21

1.4.8 set.event.videoloss	21

1.4.9 get.event.motion	21

1.4.10 set.event.motion	22

1.4.11 get.event.osd	22

1.4.12 set.event.osd	22

1.4.13 get.event.maintain_info	22

1.4.14 set.event.maintain_info	22

1.4.15 get.event.exception	22

1.4.16 set.event.exception	22

1.4.17 get.event.exception_info	23

1.5 SCHEDULE	23

1.5.1 get.schedule.recsched	23

1.5.2 set.schedule.recsched	23

1.5.3 get.schedule.motionsched	25

1.5.4 set.schedule.motionsched	25

1.5.5 get.schedule.videolosssched	30

1.5.6 set.schedule.videolosssched	31

1.5.7 get.schedule.alarminsched	32

1.5.8 set.schedule.alarminsched	32

1.5.9 get.schedule.alarmoutsched	34

1.5.10 set.schedule.alarmoutsched	34

1.5.11 get.schedule.motion_audible_sched	36

1.5.10 set.schedule.alarmoutsched	36

1.5.12 set.schedule.motion_audible_sched	36

1.5.13 get.schedule.motion_email_sched	38

1.5.10 set.schedule.alarmoutsched	38

1.5.14 set.schedule.motion_email_sched	38

1.5.15 get.schedule.motion_others	39

1.5.16 set.schedule.motion_others	39

1.5.17 get.schedule.videoloss_audible_sched	40

1.5.18 set.schedule.videoloss_audible_sched	40

1.5.19 get.schedule.videoloss_email_sched	41

1.5.20 set.schedule.videoloss_email_sched	42

1.5.21 set.schedule.videoloss_alarm_output	43

1.5.22 get.schedule.alarmin_audible_sched	43

1.5.23 set.schedule.alarmin_audible_sched	43

1.5.24 get.schedule.alarmin_email_sched	45

1.5.25 set.schedule.alarmin_email_sched	45

1.5.26 get.schedule.alarmin_ptz_sched	47

1.5.27 set.schedule.alarmin_ptz_sched	47

1.5.28 set.schedule.alarmin_alarm_output	48

1.5.29 get.schedule.motion_effective_sched	49

1.5.30 set.schedule.motion_effective_sched	49

1.5.31 get.schedule.vca_effective_sched	50

1.5.32 set.schedule.vca_effective_sched	51

1.5.33 get.schedule.vca_audible_sched	52

1.5.34 set.schedule.vca_audible_sched	53

1.5.35 get.schedule.vca_email_sched	54

1.5.36 set.schedule.vca_email_sched	54

1.5.37 get.schedule.vca_others	56

1.5.38 set.schedule.vca_others	56

1.6 PLAYBACK	56

1.6.1 get.playback.pbdayinfo	56

1.6.2 get.playback.recordinfo	56

1.6.3 get.playback.recordinfo_page	57

1.6.4 get.playback.remote	57

1.6.5 get.playback.jump	57

1.6.6 get.playback.jump_batch	58

1.6.7 get.playback.playtime	58

1.6.8 get.playback.rtspurl	58

1.6.9 set.playback.pause	58

1.6.10 set.playback.pause_batch	58

1.6.11 set.playback.snapshot	58

1.6.12 set.playback.snapshot_save	58

1.6.13 set.playback.resume	59

1.6.14 set.playback.start	59

1.6.15 set.playback.step	59

1.6.16 set.playback.step_batch	59

1.6.17 set.playback.stop	59

1.6.18 set.playback.stop_batch	59

1.6.19 set.playback.speed	59

1.6.20 set.playback.speed_batch	60

1.6.21 set.playback.close	60

1.6.22 set.playback.close_batch	60

1.7 CAMERA	60

1.7.1 get.camera.ipcparam	60

1.7.2 set.camera.ipcparam	61

1.7.3 get.camera.ipclist	61

1.7.4 add.camera.channel	62

1.7.5 modify.camera.channel	62

1.7.6 remove.camera.channel	62

1.7.7 get.camera.protocol	62

1.7.8 search.camera.chns	62

1.7.9 test.camera.ipconflict	63

1.7.10 test.camera.ipcconn	63

1.7.11 get.camera.rtsp	63

1.7.12 get.camera.imageparam	63

1.7.13 set.camera.imageparam	63

1.7.12 get.camera.imageparam	64

1.7.14 get.camera.privacy_mask	64

1.7.13 set.camera.imageparam	64

1.7.15 set.camera.privacy_mask	64

1.7.13 set.camera.imageparam	65

1.7.15 set.camera.privacy_mask	65

1.7.13 set.camera.imageparam	65

1.7.14 set.camera.privacy_mask	65

1.7.15 get.camera.privacy_mask	66

1.7.16 get.camera.ipctype	66

1.7.17 get.camera.osd	66

1.7.18 set.camera.osd	67

1.7.19 get.camera.vca.region.entarnce	67

1.7.20 set.camera.vca.region.entarnce	68

1.7.21 get.camera.vca.region.exiting	68

1.7.22 set.camera.vca.region.exiting	68

1.7.23 get.camera.vca.advanced.motion	68

1.7.24 set.camera.vca.advanced.motion	68

1.7.25 get.camera.vca.tamper.detection	68

1.7.26 set.camera.vca.tamper.detection	69

1.7.27 get.camera.vca.line.crossing	69

1.7.28 set.camera.vca.line.crossing	69

1.7.29 get.camera.vca.loitering	69

1.7.30 set.camera.vca.loitering	69

1.7.31 get.camera.vca.human.detection	70

1.7.32 set.camera.vca.human.detection	70

1.7.33 get.camera.vca.people.counting	70

1.7.34 set.camera.vca.people.counting	71

1.7.35 get.camera.vca.settings	71

1.7.36 set.camera.vca.settings	71

1.7.37 get.camera.vca.lisence	72

1.7.38 set.camera.vca.lisence	72

1.7.39 set.camera.vca.clearcount	72

1.7.40 get.camera.vca.support	72

1.7.41 get.camera.vca.support_all	72

1.7.42 get.camera.vca.person_point	72

1.8 STATUS.	73

1.8.1 get.status.allipc	73

1.8.2 get.status.ipcinfo	73

1.8.3 get.status.ipcstatus	74

1.8.4 get.status.event	74

1.8.5 get.status.ioalarm	75

1.8.6 get.status.iosensor	75

1.8.7 get.status.netbandwidth	75

1.8.8 get.status.netinfo	75

1.8.9 get.status.netspeed	76

1.9 RECORD	76

1.9.1 get.record.advanced	76

1.9.2 set.record.advanced	77

1.9.3 set.record.record	77

1.9.4 set.record.mode	77

1.9.5 get.record.schedule	77

1.9.6 set.record.schedule	77

1.9.7 set.record.batchsetting	79

1.10 PTZ	79

1.10.1 set.ptz.auto_focus	79

1.10.2 set.ptz.auto_scan	79

1.10.3 set.ptz.brush_off	79

1.10.4 set.ptz.brush_on	79

1.10.5 set.ptz.down	80

1.10.5 set.ptz.focus_in	80

1.10.6 set.ptz.focus_out	80

1.10.7 set.ptz.focus_stop	80

1.10.8 get.ptz.serial_port	80

1.10.9 set.ptz.iris_minus	80

1.10.10 set.ptz.iris_plus	80

1.10.11 set.ptz.left	81

1.10.12 set.ptz.left_down	81

1.10.13 set.ptz.left_up	81

1.10.14 set.ptz.light_off	81

1.10.15 set.ptz.light_on	81

1.10.16 set.ptz.preset_clear	81

1.10.17 set.ptz.preset_goto	81

1.10.18 set.ptz.preset_set	81

1.10.19 set.ptz.right	81

1.10.20 set.ptz.right_down	82

1.10.21 set.ptz.right_up	82

1.10.22 set.ptz.serial_port	82

1.10.23 set.ptz.stop	82

1.10.24 set.ptz.tour_add	82

1.10.25 set.ptz.tour_clear	82

1.10.26 set.ptz.tour_delete	82

1.10.27 set.ptz.tour_down	82

1.10.28 set.ptz.tour_run	83

1.10.29 set.ptz.tour_stop	83

1.10.30 set.ptz.tour_up	83

1.10.31 set.ptz.up	83

1.10.32 set.ptz.zoom_in	83

1.10.33 set.ptz.zoom_out	83

1.11 DISK	83

1.11.1 set.disk.format	83

1.11.2 get.disk.storage_info	84

1.11.3 set.disk.storage_info	84

1.11.4 set.disk.property	84

1.11.5 set.disk.delete	85

1.11.6 set.disk.recycle_mode	85

1.11.7 get.disk.recycle_mode	85

1.12 QT	85

1.11.1 set.disk.format	85

1.11.3 set.disk.storage_info	85

1.11.2 get.disk.storage_info	85

1.11.2 get.disk.storage_info	86

## Revision history

| Revision | Date | Description | Author |
| --- | --- | --- | --- |
| Draft | 2016/03/28 | Draft version | david.milesight |
| V1.0.1 | 2016/12/19 | Add remote_live_view_ex、remote_playback_ex、local_playback_ex、local_live_view_ex、tri_channels_ex Add control qt url Modify "add.camera.channel" | flycat milesight |
| V1.0.2 | 2017/05/10 | get.status.motion get.system.sysinfo | david.milesight |
| V1.0.3 | 2017/10/26 | set.schedule.motion_audible_sched get.schedule.motion_audible_sched set.schedule.motion_email_sched get.schedule.motion_email_sched set.schedule.motion_others get.schedule.motion_others set.schedule.videoloss_audible_schedget.schedule.videoloss_audible_schedset.schedule.videoloss_email_sched get.schedule.videoloss_email_sched set.schedule.videoloss_alarm_output set.schedule.alarmin_audible_sched get.schedule.alarmin_audible_sched set.schedule.alarmin_email_schedget.schedule.alarmin_email_sched set.schedule.alarmin_ptz_sched get.schedule.alarmin_ptz_sched set.schedule.alarmin_alarm_output set.camera.privacy_mask get.camera.privacy_mask set.record.batchsetting | yun.milesight |
| V1.0.4 | 2017/12/8 | test.try.camera.ipcconn get.camera.ipcimage.enhance set.camera.ipcimage.enhance get.camera.ipcimage.display set.camera.ipcimage.display get.camera.all.modelinfo get.camera.fisheye.mode set.camera.fisheye.mode | flycat milesight |
| V1.0.5 | 2018/05/22 | Update get.camera.ipcparam set.camera.ipcparam set.network.port get.network.port Add get.camera.ipctype get.camera.osd set.camera.osd get.network.p2p set.network.p2p | yun.milesight |
| V1.0.6 | 2018/10/15 | Update get.playback.pbdayinfo get.playback.rtspurl set.playback.start set.playback.resume get.playback.playtime set.playback.speed get.disk.storage_info set.disk.format Add get.playback.recordinfo get.playback.recordinfo_page set.playback.pause_batch set.playback.jump_batch set.playback.step_batch set.playback.stop_batch set.playback.speed_batch set.playback.close set.playback.close_batch set.disk.property set.disk.delete | hrz.milesight |
| V1.0.7 | 2018/10/23 | Update set.schedule.motion_effective_sched set.schedule.motion_audible_sched set.schedule.motion_email_sched set.schedule.videoloss_audible_sched set.schedule.videoloss_email_sched set.schedule.alarmin_audible_sched set.schedule.alarmin_email_sched set.schedule.recsched get.status.event get.camera.rtsp get.record.advanced set.record.advanced Add get.camera.vca.region.entarnce set.camera.vca.region.entarnce get.camera.vca.region.exiting set.camera.vca.region.exiting get.camera.vca.advanced.motion set.camera.vca.advanced.motion get.camera.vca.tamper.detection set.camera.vca.tamper.detection get.camera.vca.line.crossing set.camera.vca.line.crossing get.camera.vca.loitering set.camera.vca.loitering get.camera.vca.human.detection set.camera.vca.human.detection get.camera.vca.people.counting set.camera.vca.people.counting get.camera.vca.settings set.camera.vca.settings get.camera.vca.lisence set.camera.vca.lisence set.camera.vca.clearcount get.camera.vca.support get.camera.vca.support_all get.camera.vca.person_point get.schedule.vca_effective_sched set.schedule.vca_effective_sched get.schedule.vca_audible_sched set.schedule.vca_audible_sched get.schedule.vca_email_sched set.schedule.vca_email_sched get.schedule.vca_others set.schedule.vca_others get.schedule.motion_effective_sched set.schedule.motion_effective_sched get.event.exception_info get.disk.recycle_mode set.disk.recycle_mode | hrz.milesight |

## 1.1 NETWORK

### 1.1.1 get.network.general

| Parameter | Values | Description |
| --- | --- | --- |
| mode | int | Working mode 0 : Multi-address 1 : Load Balance 2 : Net Fault-tolerance |
| host_name | String, max 128 | Device name |
| miimon | int |  |
| bond0_primary_net | int |  |
| bond0_enable | int | Enable bond0 0 : No 1 : Yes |
| bond0_type | int | Bond type 0 : Static IP 1 : DHCP |
| bond0_ip_address | IP address | Bond0 ip |
| bond0_netmask | IP address | Bond0 netmask |
| bond0_gateway | IP address | Bond0 gateway |
| bond0_primary_dns | IP address | Bond0 primary DNS |
| bond0_second_dns | IP address | Bond0 second DNS |
| bond0_mtu | integer | Bond0 mtu |
| lan1_enable | Boolean | Enable lan1 0 : No 1 : Yes |
| lan1_type | Boolean | Lan1 type 0 : Static IP 1 : DHCP |
| lan1_ip_address | IP address | Lan1 ip |
| lan1_netmask | IP address | Lan1 netmask |
| lan1_gateway | IP address | Lan1 gateway |
| lan1_primary_dns | IP address | Lan1 primary dns |
| lan1_second_dns | IP address | Lan1 second dns |
| lan1_mtu | integer | Lan1 mtu |
| lan2_enable | Boolean | Enable lan2 0 : No 1 : Yes |
| lan2_type | Boolean | Lan1 type 0 : Static IP 1 : DHCP |
| lan2_ip_address | IP address | Lan2 ip |
| lan2_netmask | IP address | Lan2 netmask |
| lan2_gateway | IP address | Lan2 gateway |
| lan2_primary_dns | IP address | Lan2 primary dns |
| lan2_second_dns | IP address | Lan2 second dns |
| lan2_mtu | integer | Lan2 mtu |

Example: Get general parameter

http://192.168.9.129/sdk.cgi?action=get.network.general

### 1.1.2 set.network.general

To set general network parameters.

Example: Set general parameter

http://192.168.9.129/sdk.cgi?action=set.network.general&mode=0&host_name=NVR&bond0_primary_net=0&bond0_enable=1&bond0_type=1&bond0_ip_address=192.168.9.129&bond0_netmask=255.255.252.0&bond0_gateway=192.168.8.1&bond0_primary_dns=192.168.1.1&bond0_second_dns=&bond0_mtu=1500&lan1_enable=1&lan1_type=0&lan1_ip_address=192.168.9.129&lan1_netmask=255.255.252.0&lan1_gateway=192.168.8.1&lan1_primary_dns=8.8.8.8&lan1_second_dns=&lan1_mtu=1500&lan2_enable=0&lan2_type=0&lan2_ip_address=192.168.200.127&lan2_netmask=255.255.255.0&lan2_gateway=192.168.200.1&lan2_primary_dns=192.168.1.1&lan2_second_dns=&lan2_mtu=1500&lan1_dhcp_gateway=&lan2_dhcp_gateway=&tri_alarms=0

### 1.1.3 get.network.port

| Parameter | Values | Description |
| --- | --- | --- |
| enable_ssh | Boolean | Enable ssh 0 : disable 1 : enable |
| ssh_port | integer | SSH Port |
| http_port | integer | HTTP Port |
| rtsp_port | integer | RTSP Port |
| sdk_port | integer | SDK Port |
| url | String, max 12 | URL |
| url_enable | Boolean | Enable url 0 : disable 1 : enable |
| enable_push | Boolean | Enable push 0 : disable 1 : enable |
| https_port | integer | HTTPS Port |

Example: Get general parameter

http://192.168.9.129/sdk.cgi?action=get.network.port

### 1.1.4 set.network.port

To set port network parameters.

Example: Set port parameter

http://192.168.9.129/sdk.cgi?action=set.network.port&enable_ssh=1&ssh_port=22&http_port=80&rtsp_port=554&sdk_port=1100&url=192.168.9.129&url_enable=1&enable_push=0&https_port=1111

### 1.1.5 get.network.email

| Parameter | Values | Description |
| --- | --- | --- |
| username | String, max 64 | Email user name |
| password | String, max 64 | Email passwordl |
| smtp_server | String, max 64 | Smtp server address |
| port | integer | Smtp port |
| sender_addr | String, max 128 | Sender address |
| sender_name | String, max 64 | Sender name |
| enable_tls | Boolean | Enable TLS 0 : disable 1 : enable |
| enable_attach | Boolean | Enable attach 0 : disable 1 : enable |
| capture_interval | integer | Capture interval |
| receiver_id0 | integer | Receiver id |
| receiver_address0 | String, max 128 | Receiver address |
| receiver_name0 | String, max 64 | Receiver name |
| receiver_id1 | integer | Reciver id |
| receiver_address1 | String, max 128 | Receiver address |
| receiver_name1 | String, max 64 | Receiver name |
| receiver_id2 | integer | Receiver id |
| receiver_address2 | String, max 128 | Receiver address |
| receiver_name2 | String, max 64 | Receiver name |

Example: Get general parameter

http://192.168.9.129/sdk.cgi?action=get.network.email

### 1.1.6 set.network.email

To set email network parameters.

Example: Set email parameter

http://192.168.9.129/sdk.cgi?action=set.network.email&username=david&password=hws8205&smtp_server=stmp.milesight.cn&port=25&sender_addr=david@milesight.cn&sender_name=david@milesight.cn&enable_tls=1&enable_attach=0&capture_interval=0&receiver_id0=0&receiver_address0=bug@milesight.cn&receiver_name0=bug@milesight.cn&receiver_id1=0&receiver_address1=&receiver_name1=&receiver_id2=0&receiver_address2=&receiver_name2=

### 1.1.7 test.network.email

To test email network parameters.

Example: test email parameter

http://192.168.9.129/sdk.cgi?action=test.network.email

### 1.1.8 get.network.ddns

| Parameter | Values | Description |
| --- | --- | --- |
| enable | Boolean | Enable DDNS 0 : disable 1 : enable |
| domain | String, max 64 | DDNS domain |
| username | String, max 32 | DDNS username |
| password | String, max 32 | DDNS password |
| host_name | String, max 128 | DDNS host name |
| free_dns_hash | String, max 128 | DDNS hash |
| update_freq | integer | DDNS update frequency |

Example: Get general parameter

http://192.168.9.129/sdk.cgi?action=get.network.ddns

### 1.1.9 set.network.ddns

To set ddns parameters.

Example: Set ddns parameter

http://192.168.9.129/sdk.cgi?action=set.network.ddns&enable=0&domain=dyndns.org&username=david&password=david&host_name=david&free_dns_hash=&update_freq=300

### 1.1.10 get.network.pppoe

| Parameter | Values | Description |
| --- | --- | --- |
| enable | Boolean | Enable PPPoE 0 : disable 1 : enable |
| auto_connect | boolean | Auto connect 0 : disable 1 : enable |
| username | String, max 32 | PPPoE username |
| password | String, max 32 | PPPoE password |

Example: Get general parameter

http://192.168.9.129/sdk.cgi?action=get.network.pppoe

### 1.1.11 set.network.pppoe

To set pppoe parametes

Example: Set PPPoE parameter

http://192.168.9.129/sdk.cgi?action=set.network.pppoe&enable=0&auto_connect=0&username=david&password=milesight

### 1.1.12 get.network.p2p

| Parameter | Values | Description |
| --- | --- | --- |
| enable | Boolean | Enable P2P 0 : disable 1 : enable |
| p2pstatus | integer | P2P Status 0:”OK” 1:"UNAUTH": 2:"NOTFOUND" 3:"NETWORK" 4:"UNKNOWN" 5:"MAC FORMAT" 6:"AUTH FAILED" 7:"NO UUID" 8:"P2P INIT FAILED" 9:"P2P LOGIN FAILED" |

Example: Get general parameter

http://192.168.9.129/sdk.cgi?action=get.network.p2p

### 1.1.13 set.network.p2p

To set p2p status

Example: Set p2p status

http://192.168.9.129/sdk.cgi?action=set.network.p2p&enableP2P=1

## 1.2 SYSTEM

### 1.2.1 get.system.time

| Parameter | Values | Description |
| --- | --- | --- |
| ntp_enable | Boolean | Enable NTP 0 : disable 1 : enable |
| dst_enable | Boolean | Enable DST 0 : disable 1 : enable |
| time_zone | String, max 32 | Time zone |
| time_zone_name | String, max 32 | Time zone name |
| ntp_server | String, max 64 | NTP server address |
| manual_time | String, max 32 | Change current timestamp |
| time | Long | Change current timestamp |

Example: Get system parameter

http://192.168.9.129/sdk.cgi?action=get.system.time

### 1.2.2 set.system.time

To set time parametes

Example: Set Time parametes

http://192.168.9.129/sdk.cgi?action=set.system.time&ntp_enable=0&dst_enable=0&time_zone=UTC-8&time_zone_name=CHN&ntp_server=pool.ntp.org&manual_time=2016-03-06 10:37:29&time=1459910249

### 1.2.3 get.system.sysstatus

| Parameter | Values | Description |
| --- | --- | --- |
| cpu | float | Cpu utilization |
| memory | integer | memory |
| usb | integer | usb |
| cpu_temperature | integer | Cpu temperature |

Example: Get system parameter

http://192.168.9.129/sdk.cgi?action=get.system.sysstatus

### 1.2.4 get.system.sysinfo

| Parameter |  | Values | Description |
| --- | --- | --- | --- |
| hostname |  | String max 128 | Host name |
| deviceid |  | integer | Device ID |
| company |  | String max 10 | Company name |
| productno |  | String max 10 | Product NO |
| hardwareversion |  | String max 10 | Hardware version |
| softwareversion |  | String max 10 | Software version |
| oemversion |  | String max 10 | OEM version |
| devicetype |  | String max 10 | Device Type |
| devicename |  | String max 128 | Device name |
| sn |  | String max 32 | Serial number |
| guistyle |  | String max 10 | GUI style |
| reserve |  | String max 10 | reserve |
| resource | rs_max_cameras | integer | Max support camera numbers(4/8/16/32) |
|  | rs_max_analog_cameras | integer | Max support analog camera numbers(0/4/8) |
|  | rs_max_ipnc_cameras | integer | Max support IP network camera numbers |
|  | rs_camera_layout | integer |  |
|  | rs_display_num | integer | Display screen number |
|  | rs_max_sensor | integer | sensor 0~16 |
|  | rs_max_alarm | integer | Alarm 0~16 |
|  | rs_hasBuzzer | integer | Buzzer 0/1 |
|  | rs_disk_num | integer | Disk num 0~16 |
|  | rs_max_dec_chan | integer | Dec chn 0~16 |
|  | rs_max_spot_out | integer | Spot out 0~4 |
|  | rs_max_audio_in | integer | Audio in 0~16 |
|  | rs_max_audio_out | integer | Audio out |
|  | rs_max_network_lan | integer | Network LAN number(1/2) |
|  | rs_max485 | Boolean | Enable 0 : disable 1 : enable |
|  | rs_max232 | boolean | Enable 0 : disable 1 : enable |
|  | rs_has_ir | integer | IR |
| uptime |  | String max 32 | Update time |
| mac |  | String max 32 | eth0 mac |

Example: Get system parameter

http://192.168.9.129/sdk.cgi?action=get.system.sysinfo

### 1.2.5 set.system.reboot

To set system reboot parametes

Example: set system reboot parametest

http://192.168.9.129/sdk.cgi?action=set.system.reboot

### 1.2.6 set.system.devid

To set system devid parametes

Example: set system devid parametest

http://192.168.9.129/sdk.cgi?action=set.system.devid&device_id=110

### 1.2.7 set.system.shutdown

To set system shutdown parametes

Example: set system shutdown parametest

http://192.168.9.129/sdk.cgi?action=set.system.shutdown

## 1.3 USER

### 1.3.1 get.user.member

| Parameter | Values | Description |
| --- | --- | --- |
| id | Integer 0~9 | User id |
| enable | Boolean | Enable user 0 : disable 1 : enable |
| username | String, max 64 | username |
| password | String, max 32 | password |
| type | integer | User type 0 : admin 1 : operator 2 : viewer |
| permission | Integer 0~2>>31 | Local permission |
| remote_permission | Integer 0~2>>31 | Remote permission |
| local_live_view | unsigend long 0~2>>32 | Local live view channel |
| local_playback | unsigend long 0~2>>32 | Local playback channel |
| remote_live_view | unsigend long 0~2>>32 | Remote live view channel |
| remote_playback | unsigend long 0~2>>32 | Remote live view channel |
| local_live_view_ex | String, max 64 | Local live view channel |
| local_playback_ex | String, max 64 | Local playback channel |
| remote_live_view_ex | String, max 64 | Remote live view channel |
| remote_playback_ex | String, max 64 | Remote live view channel |

Example: set user parametest

http://192.168.9.129/sdk.cgi?action=get.user.member

### 1.3.2 get.user.auth

| Parameter | Values | Description |
| --- | --- | --- |
| ret | boolean | User auth result 0 : Yes 1 : No |

Example: set user parametest

http://192.168.9.129/sdk.cgi?action=get.user.auth&auth_user=admin&auth_password=a395a6211f5604092e217750e78f72c4

### 1.3.3 get.user.count

| Parameter | Values | Description |
| --- | --- | --- |
| cnt | integer | User count |

Example: set user parametest

http://192.168.9.129/sdk.cgi?action=get.user.count

### 1.3.4 set.user.member

To set user member parametes

Example: set user member

http://192.168.9.129/sdk.cgi?action=set.user.member&id=2&enable=1&username=david&password=a395a6211f5604092e217750e78f72c4&type=0&permission=-1&remote_permission=-1&local_live_view=0&local_playback=0&remote_live_view=5&remote_playback=5&local_live_view_ex=1010..&local_playback_ex=1010..&remote_live_view_ex=1010..&remote_playback=_ex=1010..

### 1.3.5 add.user.member

To add user member parametes

Example: add user member

http://192.168.9.129/sdk.cgi?action=add.user.member&id=1&enable=1&username=david&password=a395a6211f5604092e217750e78f72c4&type=0&permission=-1&remote_permission=-1&local_live_view=0&local_playback=0&remote_live_view=0&remote_playback=0&local_live_view_ex=0000.&local_playback_ex=0000..&remote_live_view_ex=0000..&remote_playback=_ex=0000..

### 1.3.6 del.user.member

To del user member parametes

Example: del user member

http://192.168.9.129/sdk.cgi?action=del.user.member&userid=1

## 1.4 EVENT

### 1.4.1 get.event.alarmin

| Parameter | Values | Description |
| --- | --- | --- |
| id | Integer 0~15 | Alarmin id |
| enable | integer | Enable alarmin 0 : Never 1 : Always 2 : By Schedule |
| name | String, max 128 | Alarmin name |
| type | integer | Open status Type 0 : normally open 1 : normally closed |
| tri_channels | An unsigned integer 0~2<<32 | trigger channels |
| tri_alarms | An unsigned integer 0~2<<32 | trigger alarms |
| buzzer_interval | interger |  |
| tri_channels_ex | String, max 64 | trigger channels |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.event.alarmin&id=0

### 1.4.2 set.event.alarmin

To set event alarmin parametes

Example: set event alarmin

http://192.168.9.129/sdk.cgi?action=set.event.alarmin&id=0&enable=1&name=david&type=0&tri_channels=1&tri_alarms=17&buzzer_interval=20&tri_channels_ex=1000..

### 1.4.3 get.event.alarmout

| Parameter | Values | Description |
| --- | --- | --- |
| id | Integer 0~3 | Alarmout id |
| name | String max 128 | Alarmout name |
| type | bolean | Alarmout type 0 : normally open 1 : normally closed |
| enable | boolean | Enable schedule 0 : Never 1 : Always 2 : By Schedule |
| duration_time | An unsigned integer | Duration time (s) |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.event.alarmout&id=0

- 1.4.4 set.event.alarmout

To set event alarmin parametes

Example: set event alarmout

http://192.168.9.129/sdk.cgi?action=set.event.alarmout&id=0&enable=1&name=david&type=0&duration_time=5

### 1.4.5 get.event.holidays

| Parameter | Values | Description |
| --- | --- | --- |
| ch | Integer 0~31 | Channel id |
| name | String max 256 | Holidays name |
| enable | boolean | Enable holidays 0 : disable 1 : enable |
| type | integer | Holidays type 0 : By date 1 : By month 2 : By week |
| start_year | integer | Start year |
| start_mon | integer | Start month |
| start_mday | integer | Start day of a month |
| start_mweek | integer | Start week of a month |
| start_wday | integer | Start day of a week |
| end_year | integer | End year |
| end_mon | integer | End month |
| end_mday | integer | End day of a month |
| end_mweek | integer | End week of a month |
| end_wday | integer | End day of a week |
| tri_channels_ex | String, max 64 | trigger channels |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.event.holidays

### 1.4.6 set.event.holidays

To set event holidays parametes

Example: set event holidays

http://192.168.9.129/sdk.cgi?action=set.event.holidays&id=0&name=Holiday&enable=0&type=1&start_year=2013&start_mon=1&start_mday=1&start_mweek=1&start_wday=0&end_year=201&end_mon=1&end_mday=1&end_mweek=1&end_wday=0

### 1.4.7 get.event.videoloss

| Parameter | Values | Description |
| --- | --- | --- |
| ch | integer | Channel id |
| enable | Integer 0 ~ 2 | Enable schedule 0 : Never 1 : Always 2 : By Schedule |
| tri_alarms | An unsigned integer | Trigger alarm output mask |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.event.videoloss&id=0

### 1.4.8 set.event.videoloss

To set event voidloss parametes

Example: set event videoloss

http://192.168.9.129/sdk.cgi/action=set.event.videoloss&id=0&enable=1&tri_alarms=0

### 1.4.9 get.event.motion

| Parameter | Values | Description |
| --- | --- | --- |
| ch | integer | Channel id |
| enable | boolean | Enable motion 0 : Never 1 : Always 2 : By Schedule |
| tri_channels | An unsigned integer | Trigger channels |
| tri_alarms | An unsigned integer | Trigger alarms |
| Mation_table | String max 356 | Motion table |
| sensitivity | An unsigned integer | sensitivity |
| buzzer_sensitivity | An unsigned integer | buzzer_sensitivity |
| ipc_sched_enable | An unsigned integer | ipc_sched_enable |
| tri_channels_ex | String, max 64 | Trigger channels |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.event.motion&id=0

### 1.4.10 set.event.motion

To set event voidloss parametes

Example: set event motion

http://192.168.9.129/sdk.cgi?action=set.event.motion&id=0&enable=1&tri_alarms=5&tri_channels=20&motion_table=0000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000&sensitivity=0&buzzer_interval=20&ipc_sched_enable=0&tri_channels_ex=000...

### 1.4.11 get.event.osd

| Parameter | Values | Description |
| --- | --- | --- |
| ch[i] | integer | Channel id |
| name[i] | String max 64 | Osd name |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.event.osd

### 1.4.12 set.event.osd

To set event ptzinfo parametes

Example: set event osd parametes

http://192.168.9.129/sdk.cgi?action=set.event.osd&id=0&name=david milesight

### 1.4.13 get.event.maintain_info

| Parameter | Values | Description |
| --- | --- | --- |
| log | integer | Save log time (month) |
| photo | integer | Save photo time(month) |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.event.maintain_info

### 1.4.14 set.event.maintain_info

To set event maintain info parametes

Example: set event maintain info parametes

http://192.168.9.129/sdk.cgi?action=set.event.maintain_info&log=6&photo=6

### 1.4.15 get.event.exception

| Parameter | Values | Description |
| --- | --- | --- |
| network_disconn | integer | Network disconnected |
| disk_full | integer | HDD full |
| record_fail | integer | Record failed |
| disk_fail | integer | HDD failed |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.event.exception

### 1.4.16 set.event.exception

To set event exception parametes

Example: set event exception parametes

http://192.168.9.129/sdk.cgi?action=set.event.exception&network_disconn=0&disk_full=0&record_fail=0&disk_fail=1

### 1.4.17 get.event.exception_info

| Parameter | Values | Description |
| --- | --- | --- |
| cnt | integer | Exception information count |
| event | string | Event |
| information | string | Information |

Example: get event exception information

http://192.168.9.129/sdk.cgi?action=get.event.exception_info

## 1.5 SCHEDULE

### 1.5.1 get.schedule.recsched

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] Wholeday_enable : 0/1 [integer] Wholeday_action_type : wholeday action type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
|  | wholeday_enable |  |  |  |
|  | wholeday_action_type |  |  |  |
| enable |  |  | integer | ~ |
| ch |  |  | integer | Channel id |

Example: get schedule parametes

http://192.168.9.129/sdk.cgi?action=get.schedule.recsched&ch=0

### 1.5.2 set.schedule.recsched

To set schedule recsched parametes

Example: set schedule recsched parametes

http://192.168.9.129/sdk.cgi?action=set.schedule.recsched&ch=0&enable=1&copy_channel=1001100000000000000000000000000000000000000000000000000000000000&wholeday_enable[0]=1&wholeday_action_type[0]=1&start_time[0][0]=00:00&end_time[0][0]=01:00&action_type[0][0]=0&start_time[0][1]=00:00&end_time[0][1]=12:00&action_type[0][1]=1&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=0&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&wholeday_enable[1]=0&wholeday_action_type[1]=0&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&wholeday_enable[2]=0&wholeday_action_type[2]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&wholeday_enable[3]=0&wholeday_action_type[3]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&wholeday_enable[4]=0&wholeday_action_type[4]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&wholeday_enable[5]=0&wholeday_action_type[5]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&wholeday_enable[6]=0&wholeday_action_type[6]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&wholeday_enable[7]=0&wholeday_action_type[7]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

### 1.5.3 get.schedule.motionsched

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] Wholeday_enable : 0/1 [integer] Wholeday_action_type : wholeday action type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
|  | wholeday_enable |  |  |  |
|  | wholeday_action_type |  |  |  |
| id |  |  | integer | Channel id |
| copy_channel |  |  | String 64 | 0：not need to copy 1：need to copy |

Example: get schedule parametes

http://192.168.9.129/sdk.cgi?action=get.schedule.motionsched&ch=0

### 1.5.4 set.schedule.motionsched

To set schedule motionsched parametes

Example: set schedule motionsched parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.motionsched&ch=0&sensitivity=5&enableMS=1&triAlarms=63&triChannels=11111111111111111111111111111111&motionTable=11111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111&wholeday_enable[0]=1&wholeday_action_type[0]=1&start_time[0][0]=00:00&end_time[0][0]=01:00&action_type[0][0]=2&start_time[0][1]=03:00&end_time[0][1]=12:00&action_type[0][1]=2&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=0&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&wholeday_enable[1]=0&wholeday_action_type[1]=0&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&wholeday_enable[2]=0&wholeday_action_type[2]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&wholeday_enable[3]=0&wholeday_action_type[3]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&wholeday_enable[4]=0&wholeday_action_type[4]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&wholeday_enable[5]=0&wholeday_action_type[5]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&wholeday_enable[6]=0&wholeday_action_type[6]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&wholeday_enable[7]=0&wholeday_action_type[7]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0&audible_enable=2&audible_interval=20&start_time_audible[0][0]=00:00&end_time_audible[0][0]=24:00&action_type_audible[0][0]=8&start_time_audible[0][1]=00:00&end_time_audible[0][1]=12:00&action_type_audible[0][1]=1&start_time_audible[0][2]=&end_time_audible[0][2]=&action_type_audible[0][2]=0&start_time_audible[0][3]=&end_time_audible[0][3]=&action_type_audible[0][3]=0&start_time_audible[0][4]=&end_time_audible[0][4]=&action_type_audible[0][4]=0&start_time_audible[0][5]=&end_time_audible[0][5]=&action_type_audible[0][5]=0&start_time_audible[0][6]=&end_time_audible[0][6]=&action_type_audible[0][6]=0&start_time_audible[0][7]=&end_time_audible[0][7]=&action_type_audible[0][7]=0&start_time_audible[0][8]=&end_time_audible[0][8]=&action_type_audible[0][8]=0&start_time_audible[0][9]=&end_time_audible[0][9]=&action_type_audible[0][9]=0&start_time_audible[0][10]=&end_time_audible[0][10]=&action_type_audible[0][10]=0&start_time_audible[0][11]=12:00&end_time_audible[0][11]=14:00&action_type_audible[0][11]=2&start_time_audible[1][0]=&end_time_audible[1][0]=&action_type_audible[1][0]=0&start_time_audible[1][1]=&end_time_audible[1][1]=&action_type_audible[1][1]=0&start_time_audible[1][2]=&end_time_audible[1][2]=&action_type_audible[1][2]=0&start_time_audible[1][3]=&end_time_audible[1][3]=&action_type_audible[1][3]=0&start_time_audible[1][4]=&end_time_audible[1][4]=&vaction_type[1][4]=0&start_time_audible[1][5]=&end_time_audible[1][5]=&action_type_audible[1][5]=0&start_time_audible[1][6]=&end_time_audible[1][6]=&action_type_audible[1][6]=0&start_time_audible[1][7]=&end_time_audible[1][7]=&action_type_audible[1][7]=0&start_time_audible[1][8]=&end_time_audible[1][8]=&action_type_audible[1][8]=0&start_time_audible[1][9]=&end_time_audible[1][9]=&action_type_audible[1][9]=0&start_time_audible[1][10]=&end_time_audible[1][10]=&action_type_audible[1][10]=0&start_time_audible[1][11]=&end_time_audible[1][11]=&action_type_audible[1][11]=0&start_time_audible[2][0]=&end_time_audible[2][0]=&action_type_audible[2][0]=0&start_time_audible[2][1]=&end_time_audible[2][1]=&action_type_audible[2][1]=0&start_time_audible[2][2]=&end_time_audible[2][2]=&action_type_audible[2][2]=0&start_time_audible[2][3]=&end_time_audible[2][3]=&action_type_audible[2][3]=0&start_time_audible[2][4]=&end_time_audible[2][4]=&action_type_audible[2][4]=0&start_time_audible[2][5]=&end_time_audible[2][5]=&action_type_audible[2][5]=0&start_time_audible[2][6]=&end_time_audible[2][6]=&action_type_audible[2][6]=0&start_time_audible[2][7]=&end_time_audible[2][7]=&action_type_audible[2][7]=0&start_time_audible[2][8]=&end_time_audible[2][8]=&action_type_audible[2][8]=0&start_time_audible[2][9]=&end_time_audible[2][9]=&action_type_audible[2][9]=0&vstart_time[2][10]=&end_time_audible[2][10]=&action_type_audible[2][10]=0&start_time_audible[2][11]=&end_time_audible[2][11]=&action_type_audible[2][11]=0&start_time_audible[3][0]=&end_time_audible[3][0]=&action_type_audible[3][0]=0&start_time_audible[3][1]=&end_time_audible[3][1]=&action_type_audible[3][1]=0&start_time_audible[3][2]=&end_time_audible[3][2]=&action_type_audible[3][2]=0&start_time_audible[3][3]=&end_time_audible[3][3]=&action_type_audible[3][3]=0&start_time_audible[3][4]=&end_time_audible[3][4]=&action_type_audible[3][4]=0&start_time_audible[3][5]=&end_time_audible[3][5]=&action_type_audible[3][5]=0&start_time_audible[3][6]=&end_time_audible[3][6]=&action_type_audible[3][6]=0&start_time_audible[3][7]=&end_time_audible[3][7]=&action_type_audible[3][7]=0&start_time_audible[3][8]=&end_time_audible[3][8]=&action_type_audible[3][8]=0&start_time_audible[3][9]=&end_time_audible[3][9]=&action_type_audible[3][9]=0&start_time_audible[3][10]=&end_time_audible[3][10]=&action_type_audible[3][10]=0&start_time_audible[3][11]=&end_time_audible[3][11]=&action_type_audible[3][11]=0&start_time_audible[4][0]=&end_time_audible[4][0]=&action_type_audible[4][0]=0&start_time_audible[4][1]=&end_time_audible[4][1]=&action_type_audible[4][1]=0&start_time_audible[4][2]=&end_time_audible[4][2]=&action_type_audible[4][2]=0&start_time_audible[4][3]=&end_time_audible[4][3]=&action_type_audible[4][3]=0&start_time_audible[4][4]=&end_time_audible[4][4]=&action_type_audible[4][4]=0&start_time_audible[4][5]=&end_time_audible[4][5]=&action_type_audible[4][5]=0&start_time_audible[4][6]=&end_time_audible[4][6]=&action_type_audible[4][6]=0&start_time_audible[4][7]=&end_time_audible[4][7]=&action_type_audible[4][7]=0&start_time_audible[4][8]=&end_time_audible[4][8]=&action_type_audible[4][8]=0&start_time_audible[4][9]=&end_time_audible[4][9]=&action_type_audible[4][9]=0&start_time_audible[4][10]=&end_time_audible[4][10]=&action_type_audible[4][10]=0&start_time_audible[4][11]=&end_time_audible[4][11]=&action_type_audible[4][11]=0&start_time_audible[5][0]=&end_time_audible[5][0]=&action_type_audible[5][0]=0&start_time_audible[5][1]=&end_time_audible[5][1]=&action_type_audible[5][1]=0&start_time_audible[5][2]=&end_time_audible[5][2]=&action_type_audible[5][2]=0&start_time_audible[5][3]=&end_time_audible[5][3]=&action_type_audible[5][3]=0&start_time_audible[5][4]=&end_time_audible[5][4]=&action_type_audible[5][4]=0&start_time_audible[5][5]=&end_time_audible[5][5]=&action_type_audible[5][5]=0&start_time_audible[5][6]=&end_time_audible[5][6]=&action_type_audible[5][6]=0&start_time_audible[5][7]=&end_time_audible[5][7]=&action_type_audible[5][7]=0&start_time_audible[5][8]=&end_time_audible[5][8]=&action_type_audible[5][8]=0&start_time_audible[5][9]=&end_time_audible[5][9]=&action_type_audible[5][9]=0&start_time_audible[5][10]=&end_time_audible[5][10]=&action_type_audible[5][10]=0&start_time_audible[5][11]=&end_time_audible[5][11]=&action_type_audible[5][11]=0&start_time_audible[6][0]=&end_time_audible[6][0]=&action_type_audible[6][0]=0&start_time_audible[6][1]=&end_time_audible[6][1]=&action_type_audible[6][1]=0&start_time_audible[6][2]=&end_time_audible[6][2]=&action_type_audible[6][2]=0&start_time_audible[6][3]=&end_time_audible[6][3]=&action_type_audible[6][3]=0&start_time_audible[6][4]=&end_time_audible[6][4]=&action_type_audible[6][4]=0&start_time_audible[6][5]=&end_time_audible[6][5]=&action_type_audible[6][5]=0&start_time_audible[6][6]=&end_time_audible[6][6]=&action_type_audible[6][6]=0&start_time_audible[6][7]=&end_time_audible[6][7]=&action_type_audible[6][7]=0&start_time_audible[6][8]=&end_time_audible[6][8]=&action_type_audible[6][8]=0&start_time_audible[6][9]=&end_time_audible[6][9]=&action_type_audible[6][9]=0&start_time_audible[6][10]=&end_time_audible[6][10]=&action_type_audible[6][10]=0&start_time_audible[6][11]=&end_time_audible[6][11]=&action_type_audible[6][11]=0&start_time_audible[7][0]=&end_time_audible[7][0]=&action_type_audible[7][0]=0&start_time_audible[7][1]=&end_time_audible[7][1]=&action_type_audible[7][1]=0&start_time_audible[7][2]=&end_time_audible[7][2]=&action_type_audible[7][2]=0&start_time_audible[7][3]=&end_time_audible[7][3]=&action_type_audible[7][3]=0&start_time_audible[7][4]=&end_time_audible[7][4]=&action_type_audible[7][4]=0&start_time_audible[7][5]=&end_time_audible[7][5]=&action_type_audible[7][5]=0&start_time_audible[7][6]=&end_time_audible[7][6]=&action_type_audible[7][6]=0&start_time_audible[7][7]=&end_time_audible[7][7]=&action_type_audible[7][7]=0&start_time_audible[7][8]=&end_time_audible[7][8]=&action_type_audible[7][8]=0&start_time_audible[7][9]=&end_time_audible[7][9]=&action_type_audible[7][9]=0&start_time_audible[7][10]=&end_time_audible[7][10]=&action_type_audible[7][10]=0&start_time_audible[7][11]=&end_time_audible[7][11]=&action_type_audible[7][11]=0&email_enable=0&email_interval=60&start_time_email[0][0]=00:00&end_time_email[0][0]=24:00&action_type_email[0][0]=8&start_time_email[0][1]=00:00&end_time_email[0][1]=00:00&action_type_email[0][1]=2&start_time_email[0][2]=&end_time_email[0][2]=&action_type_emailaction_type_email[0][2]=2&start_time_email[0][3]=&end_time_email[0][3]=&action_type_email[0][3]=0&start_time_email[0][4]=&end_time_email[0][4]=&action_type_email[0][4]=0&start_time_email[0][5]=&end_time_email[0][5]=&action_type_email[0][5]=0&start_time_email[0][6]=&end_time_email[0][6]=&action_type_email[0][6]=0&start_time_email[0][7]=&end_time_email[0][7]=&action_type_email[0][7]=0&start_time_email[0][8]=&end_time_email[0][8]=&action_type_email[0][8]=0&start_time_email[0][9]=&end_time_email[0][9]=&action_type_email[0][9]=0&start_time_email[0][10]=&end_time_email[0][10]=&action_type_email[0][10]=0&start_time_email[0][11]=12:00&end_time_email[0][11]=14:00&action_type_email[0][11]=2&start_time_email[1][0]=&end_time_email[1][0]=&action_type_email[1][0]=0&start_time_email[1][1]=&end_time_email[1][1]=&action_type_email[1][1]=0&start_time_email[1][2]=&end_time_email[1][2]=&action_type_email[1][2]=0&start_time_email[1][3]=&end_time_email[1][3]=&action_type_email[1][3]=0&start_time_email[1][4]=&end_time_email[1][4]=&action_type_email[1][4]=0&start_time_email[1][5]=&end_time_email[1][5]=&action_type_email[1][5]=0&start_time_email[1][6]=&end_time_email[1][6]=&action_type_email[1][6]=0&start_time_email[1][7]=&end_time_email[1][7]=&action_type_email[1][7]=0&start_time_email[1][8]=&end_time_email[1][8]=&action_type_email[1][8]=0&start_time_email[1][9]=&end_time_email[1][9]=&action_type_email[1][9]=0&start_time_email[1][10]=&end_time_email[1][10]=&action_type_email[1][10]=0&start_time_email[1][11]=&end_time_email[1][11]=&action_type_email[1][11]=0&start_time_email[2][0]=&end_time_email[2][0]=&action_type_email[2][0]=0&start_time_email[2][1]=&end_time_email[2][1]=&action_type_email[2][1]=0&start_time_email[2][2]=&end_time_email[2][2]=&action_type_email[2][2]=0&start_time_email[2][3]=&end_time_email[2][3]=&action_type_email[2][3]=0&start_time_email[2][4]=&end_time_email[2][4]=&action_type_email[2][4]=0&start_time_email[2][5]=&end_time_email[2][5]=&action_type_email[2][5]=0&start_time_email[2][6]=&end_time_email[2][6]=&action_type_email[2][6]=0&start_time_email[2][7]=&end_time_email[2][7]=&action_type_email[2][7]=0&start_time_email[2][8]=&end_time_email[2][8]=&action_type_email[2][8]=0&start_time_email[2][9]=&end_time_email[2][9]=&action_type_email[2][9]=0&start_time_email[2][10]=&end_time_email[2][10]=&action_type_email[2][10]=0&start_time_email[2][11]=&end_time_email[2][11]=&action_type_email[2][11]=0&start_time_email[3][0]=&end_time_email[3][0]=&action_type_email[3][0]=0&start_time_email[3][1]=&end_time_email[3][1]=&action_type_email[3][1]=0&start_time_email[3][2]=&end_time_email[3][2]=&action_type_email[3][2]=0&start_time_email[3][3]=&end_time_email[3][3]=&action_type_email[3][3]=0&start_time_email[3][4]=&end_time_email[3][4]=&action_type_email[3][4]=0&start_time_email[3][5]=&end_time_email[3][5]=&action_type_email[3][5]=0&start_time_email[3][6]=&end_time_email[3][6]=&action_type_email[3][6]=0&start_time_email[3][7]=&end_time_email[3][7]=&action_type_email[3][7]=0&start_time_email[3][8]=&end_time_email[3][8]=&action_type_email[3][8]=0&start_time_email[3][9]=&end_time_email[3][9]=&action_type_email[3][9]=0&start_time_email[3][10]=&end_time_email[3][10]=&action_type_email[3][10]=0&start_time_email[3][11]=&end_time_email[3][11]=&action_type_email[3][11]=0&start_time_email[4][0]=&end_time_email[4][0]=&action_type_email[4][0]=0&start_time_email[4][1]=&end_time_email[4][1]=&action_type_email[4][1]=0&start_time_email[4][2]=&end_time_email[4][2]=&action_type_email[4][2]=0&start_time_email[4][3]=&end_time_email[4][3]=&action_type_email[4][3]=0&start_time_email[4][4]=&end_time_email[4][4]=&action_type_email[4][4]=0&start_time_email[4][5]=&end_time_email[4][5]=&action_type_email[4][5]=0&start_time_email[4][6]=&end_time_email[4][6]=&action_type_email[4][6]=0&start_time_email[4][7]=&end_time_email[4][7]=&action_type_email[4][7]=0&start_time_email[4][8]=&end_time_email[4][8]=&action_type_email[4][8]=0&start_time_email[4][9]=&end_time_email[4][9]=&action_type_email[4][9]=0&start_time_email[4][10]=&end_time_email[4][10]=&action_type_email[4][10]=0&start_time_email[4][11]=&end_time_email[4][11]=&action_type_email[4][11]=0&start_time_email[5][0]=&end_time_email[5][0]=&action_type_email[5][0]=0&start_time_email[5][1]=&end_time_email[5][1]=&action_type_email[5][1]=0&start_time_email[5][2]=&end_time_email[5][2]=&action_type_email[5][2]=0&start_time_email[5][3]=&end_time_email[5][3]=&action_type_email[5][3]=0&start_time_email[5][4]=&end_time_email[5][4]=&action_type_email[5][4]=0&start_time_email[5][5]=&end_time_email[5][5]=&action_type_email[5][5]=0&start_time_email[5][6]=&end_time_email[5][6]=&action_type_email[5][6]=0&start_time_email[5][7]=&end_time_email[5][7]=&action_type_email[5][7]=0&start_time_email[5][8]=&end_time_email[5][8]=&action_type_email[5][8]=0&start_time_email[5][9]=&end_time_email[5][9]=&action_type_email[5][9]=0&start_time_email[5][10]=&end_time_email[5][10]=&action_type_email[5][10]=0&start_time_email[5][11]=&end_time_email[5][11]=&action_type_email[5][11]=0&start_time_email[6][0]=&end_time_email[6][0]=&action_type_email[6][0]=0&start_time_email[6][1]=&end_time_email[6][1]=&action_type_email[6][1]=0&start_time_email[6][2]=&end_time_email[6][2]=&action_type_email[6][2]=0&start_time_email[6][3]=&end_time_email[6][3]=&action_type_email[6][3]=0&start_time_email[6][4]=&end_time_email[6][4]=&action_type_email[6][4]=0&start_time_email[6][5]=&end_time_email[6][5]=&action_type_email[6][5]=0&start_time_email[6][6]=&end_time_email[6][6]=&action_type_email[6][6]=0&start_time_email[6][7]=&end_time_email[6][7]=&action_type_email[6][7]=0&start_time_email[6][8]=&end_time_email[6][8]=&action_type_email[6][8]=0&start_time_email[6][9]=&end_time_email[6][9]=&action_type_email[6][9]=0&start_time_email[6][10]=&end_time_email[6][10]=&action_type_email[6][10]=0&start_time_email[6][11]=&end_time_email[6][11]=&action_type_email[6][11]=0&start_time_email[7][0]=&end_time_email[7][0]=&action_type_email[7][0]=0&start_time_email[7][1]=&end_time_email[7][1]=&action_type_email[7][1]=0&start_time_email[7][2]=&end_time_email[7][2]=&action_type_email[7][2]=0&start_time_email[7][3]=&end_time_email[7][3]=&action_type_email[7][3]=0&start_time_email[7][4]=&end_time_email[7][4]=&action_type_email[7][4]=0&start_time_email[7][5]=&end_time_email[7][5]=&action_type_email[7][5]=0&start_time_email[7][6]=&end_time_email[7][6]=&action_type_email[7][6]=0&start_time_email[7][7]=&end_time_email[7][7]=&action_type_email[7][7]=0&start_time_email[7][8]=&end_time_email[7][8]=&action_type_email[7][8]=0&start_time_email[7][9]=&end_time_email[7][9]=&action_type_email[7][9]=0&start_time_email[7][10]=&end_time_email[7][10]=&action_type_email[7][10]=0&start_time_email[7][11]=&end_time_email[7][11]=&action_type_email[7][11]=0

### 1.5.5 get.schedule.videolosssched

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] Wholeday_enable : 0/1 [integer] Wholeday_action_type : wholeday action type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
|  | wholeday_enable |  |  |  |
|  | wholeday_action_type |  |  |  |
| id |  |  | integer | Channel id |

Example: get schedule parametes

http://192.168.9.129/sdk.cgi?action=get.schedule.videolosssched&ch=0

### 1.5.6 set.schedule.videolosssched

To set schedule videolosssched parametes

Example: set schedule videolosssched parametes

http://192.168.9.129/sdk.cgi?action=set.schedule.videolosssched&ch=0&wholeday_enable[0]=1&wholeday_action_type[0]=1&start_time[0][0]=00:00&end_time[0][0]=01:00&action_type[0][0]=0&start_time[0][1]=00:00&end_time[0][1]=12:00&action_type[0][1]=1&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=0&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&wholeday_enable[1]=0&wholeday_action_type[1]=0&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&wholeday_enable[2]=0&wholeday_action_type[2]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&wholeday_enable[3]=0&wholeday_action_type[3]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&wholeday_enable[4]=0&wholeday_action_type[4]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&wholeday_enable[5]=0&wholeday_action_type[5]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&wholeday_enable[6]=0&wholeday_action_type[6]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&wholeday_enable[7]=0&wholeday_action_type[7]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

### 1.5.7 get.schedule.alarminsched

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] Wholeday_enable : 0/1 [integer] Wholeday_action_type : wholeday action type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
|  | wholeday_enable |  |  |  |
|  | wholeday_action_type |  |  |  |
| id |  |  | integer | Channel id |

Example: get schedule parametes

http://192.168.9.129/sdk.cgi?action=get.schedule.alarminsched&ch=0

### 1.5.8 set.schedule.alarminsched

To set schedule alarminsched parametes

Example: set schedule alrminsched parametes

http://192.168.9.129/sdk.cgi?action=set.schedule.alarminsched&ch=0&wholeday_enable[0]=1&wholeday_action_type[0]=1&start_time[0][0]=00:00&end_time[0][0]=01:00&action_type[0][0]=0&start_time[0][1]=00:00&end_time[0][1]=12:00&action_type[0][1]=1&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=0&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&wholeday_enable[1]=0&wholeday_action_type[1]=0&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&wholeday_enable[2]=0&wholeday_action_type[2]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&wholeday_enable[3]=0&wholeday_action_type[3]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&wholeday_enable[4]=0&wholeday_action_type[4]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&wholeday_enable[5]=0&wholeday_action_type[5]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&wholeday_enable[6]=0&wholeday_action_type[6]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&wholeday_enable[7]=0&wholeday_action_type[7]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

### 1.5.9 get.schedule.alarmoutsched

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] Wholeday_enable : 0/1 [integer] Wholeday_action_type : wholeday action type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
|  | wholeday_enable |  |  |  |
|  | wholeday_action_type |  |  |  |
| id |  |  | integer | Channel id |

Example: get schedule parametes

http://192.168.9.129/sdk.cgi?action=get.schedule.alarmoutsched&ch=0

### 1.5.10 set.schedule.alarmoutsched

To set schedule alarmoutsched parametes

Example: set schedule alrmoutsched parametes

http://192.168.9.129/sdk.cgi?action=set.schedule.alarmoutsched&ch=0&wholeday_enable[0]=1&wholeday_action_type[0]=1&start_time[0][0]=00:00&end_time[0][0]=01:00&action_type[0][0]=0&start_time[0][1]=00:00&end_time[0][1]=12:00&action_type[0][1]=1&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=0&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&wholeday_enable[1]=0&wholeday_action_type[1]=0&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&wholeday_enable[2]=0&wholeday_action_type[2]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&wholeday_enable[3]=0&wholeday_action_type[3]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&wholeday_enable[4]=0&wholeday_action_type[4]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&wholeday_enable[5]=0&wholeday_action_type[5]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&wholeday_enable[6]=0&wholeday_action_type[6]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&wholeday_enable[7]=0&wholeday_action_type[7]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
| ch |  |  | integer | Channel id |

### 1.5.11 get.schedule.motion_audible_sched

Example: get schedule parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.motion_audible_sched&ch=0

### 1.5.10 set.schedule.alarmoutsched

To set schedule motion_audible_sched parametes

### 1.5.12 set.schedule.motion_audible_sched

Example: set schedule motion_audible_sched parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.motion_audible_sched&ch=0&audible_enable=2&audibleinterval=20&start_time[0][0]=00:00&end_time[0][0]=24:00&action_type[0][0]=8&start_time[0][1]=00:00&end_time[0][1]=12:00&action_type[0][1]=1&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=0&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
| ch |  |  | integer | Channel id |

### 1.5.13 get.schedule.motion_email_sched

Example: get schedule parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.motion_email_sched&ch=0

### 1.5.10 set.schedule.alarmoutsched

To set schedule motion_email_sched parametes

### 1.5.14 set.schedule.motion_email_sched

Example: set schedule motion_email_sched parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.motion_email_sched&ch=0&email_enable=0&email_interval=0&start_time[0][0]=00:00&end_time[0][0]=24:00&action_type[0][0]=2&start_time[0][1]=00:00&end_time[0][1]=00:00&action_type[0][1]=2&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=2&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

| Parameter | Values | Description |
| --- | --- | --- |
| type | integer | triAlarms:2 triChannels:1 |
| ch | integer | Channel id |
| triAlarms | integer | triAlarms:Alarm Input rec |
| triChannels | char | triChannels:Trigger Channel rec [char [64]] |

### 1.5.15 get.schedule.motion_others

Example: get others parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.motion_others&ch=0

To set schedule motion_others parametes

### 1.5.16 set.schedule.motion_others

Example: set schedule motion_others parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.motion_others&type=2&ch=0&triAlarms=63

http://192.168.9.147/sdk.cgi?action=set.schedule.motion_others&type=1&ch=0&triChannels=11111111111111111111111111111111

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
| ch |  |  | integer | Channel id |

### 1.5.17 get.schedule.videoloss_audible_sched

Example: get schedule parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.videoloss_audible_sched&ch=0

To set schedule videoloss_audible_sched parametes

### 1.5.18 set.schedule.videoloss_audible_sched

Example: set schedule videoloss_audible_sched parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.videoloss_audible_sched&ch=0&enable=0&interval=0&copy_channel=1001100000000000000000000000000000000000000000000000000000000000&start_time[0][0]=00:00&end_time[0][0]=24:00&action_type[0][0]=9&start_time[0][1]=00:00&end_time[0][1]=00:00&action_type[0][1]=9&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=9&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
| ch |  |  | integer | Channel id |
| copy_channel |  |  | string 64 | 0：not need to copy 1：need to copy |

### 1.5.19 get.schedule.videoloss_email_sched

Example: get schedule parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.videoloss_email_sched&ch=0

To set schedule videoloss_email_sched parametes

### 1.5.20 set.schedule.videoloss_email_sched

Example: set schedule videoloss_email_sched parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.videoloss_email_sched&ch=0&enable=0&interval=0&copy_channel=1001100000000000000000000000000000000000000000000000000000000000&start_time[0][0]=00:00&end_time[0][0]=24:00&action_type[0][0]=9&start_time[0][1]=00:00&end_time[0][1]=00:00&action_type[0][1]=9&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=9&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

| Parameter | Values | Description |
| --- | --- | --- |
| ch | integer | Channel id |
| triAlarms | integer | triAlarms:Alarm Input rec |

### 1.5.21 set.schedule.videoloss_alarm_output

To set schedule videoloss_alarm_output parametes

Example: set schedule videoloss_alarm_output parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.videoloss_alarm_output&ch=0&triAlarms=63

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
| ch |  |  | integer | Channel id |
| copy_channel |  |  | String 64 | 0：not need to copy 1：need to copy |

### 1.5.22 get.schedule.alarmin_audible_sched

Example: get schedule parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.alarmin_audible_sched&ch=0

To set schedule alarmin_audible_sched parametes

### 1.5.23 set.schedule.alarmin_audible_sched

Example: set schedule alarmin_audible_sched parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.alarmin_audible_sched&ch=0&enable=0&interval=0&copy_channel=1001100000000000000000000000000000000000000000000000000000000000&start_time[0][0]=00:00&end_time[0][0]=24:00&action_type[0][0]=6&start_time[0][1]=00:00&end_time[0][1]=00:00&action_type[0][1]=6&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=6&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=6&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=6&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=6&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
| ch |  |  | integer | Channel id |
| copy_channel |  |  | String 64 | 0：not need to copy 1：need to copy |

### 1.5.24 get.schedule.alarmin_email_sched

Example: get schedule parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.alarmin_email_sched&ch=0

To set schedule alarmin_email_sched parametes

### 1.5.25 set.schedule.alarmin_email_sched

Example: set schedule alarmin_email_sched parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.alarmin_email_sched&ch=0&enable=0&interval=0&copy_channel=1001100000000000000000000000000000000000000000000000000000000000&start_time[0][0]=00:00&end_time[0][0]=24:00&action_type[0][0]=6&start_time[0][1]=00:00&end_time[0][1]=00:00&action_type[0][1]=6&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=6&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=6&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=6&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=6&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
| ch |  |  | integer | Channel id |
| copy_channel |  |  | String 64 | 0：not need to copy 1：need to copy |

### 1.5.26 get.schedule.alarmin_ptz_sched

Example: get schedule parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.alarmin_ptz_sched&ch=0

To set schedule alarmin_ptz_sched parametes

### 1.5.27 set.schedule.alarmin_ptz_sched

Example: set schedule alarmin_ptz_sched parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.alarmin_ptz_sched&ch=2&acto_ptz_channel=2&acto_ptz_preset_enable=1&acto_ptz_preset=1&acto_ptz_patrol_enable=0&acto_ptz_patrol=0&start_time[0][0]=00:00&end_time[0][0]=01:00&action_type[0][0]=0&start_time[0][1]=00:00&end_time[0][1]=12:00&action_type[0][1]=6&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=6&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=6&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=6&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=6&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

| Parameter | Values | Description |
| --- | --- | --- |
| type | integer | triAlarms:1 triChannels:0 |
| ch | integer | Channel id |
| triAlarms | integer | triAlarms:Alarm Input rec |
| triChannels | char | triChannels:Trigger Channel rec [char [64]] |

### 1.5.28 set.schedule.alarmin_alarm_output

To set schedule alarmin_alarm_output parametes

Example: set schedule alarmin_alarm_output parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.alarmin_alarm_output&ch=0&triAlarms=63&type=1

http://192.168.9.147/sdk.cgi?action=set.schedule.alarmin_alarm_output&ch=0&triChannels=11111111111111111111111111111111&type=0

### 1.5.29 get.schedule.motion_effective_sched

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
| ch |  |  | integer | Channel id |
| copy_channel |  |  | String 64 | 0：not need to copy 1：need to copy |

Example: get schedule parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.motion_effective_sched&ch=0

### 1.5.30 set.schedule.motion_effective_sched

http://192.168.9.147/sdk.cgi?action=set.schedule.motion_effective_sched&ch=0&copy_channel=1001100000000000000000000000000000000000000000000000000000000000&start_time[0][0]=00:00&end_time[0][0]=24:00&action_type[0][0]=6&start_time[0][1]=00:00&end_time[0][1]=00:00&action_type[0][1]=6&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=6&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=6&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=6&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=6&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

### 1.5.31 get.schedule.vca_effective_sched

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
| ch |  |  | integer | Channel id |
| event |  |  | integer | Smart event type 0:region entarnce 1:region exiting 2:advanced.motion detection 3:tamper detection 4:line crossing 5:loitering 6:human detection 7:people counting |

Example: get schedule parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.vca_effective_sched&ch=0&event=0

### 1.5.32 set.schedule.vca_effective_sched

Example: set schedule vca effective sched parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.vca_effective_sched&ch=0&event=0&&start_time[0][0]=00:00&end_time[0][0]=24:00&action_type[0][0]=8&start_time[0][1]=00:00&end_time[0][1]=12:00&action_type[0][1]=1&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=0&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

### 1.5.33 get.schedule.vca_audible_sched

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
| ch |  |  | integer | Channel id |
| event |  |  | integer | Smart event type 0:region entarnce 1:region exiting 2:advanced.motion detection 3:tamper detection 4:line crossing 5:loitering 6:human detection 7:people counting |
| interval |  |  | integer | Triggered Interval(s) |

Example: get schedule parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.vca_audible_sched&ch=0&event=0

### 1.5.34 set.schedule.vca_audible_sched

Example: set schedule vca audible sched parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.vca_audible_sched&ch=0&event=0&interval=20&&start_time[0][0]=00:00&end_time[0][0]=24:00&action_type[0][0]=8&start_time[0][1]=00:00&end_time[0][1]=12:00&action_type[0][1]=1&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=0&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

### 1.5.35 get.schedule.vca_email_sched

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
| ch |  |  | integer | Channel id |
| event |  |  | integer | Smart event type 0:region entarnce 1:region exiting 2:advanced.motion detection 3:tamper detection 4:line crossing 5:loitering 6:human detection 7:people counting |
| interval |  |  | integer | Triggered Interval(s) |

Example: get schedule parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.vca_email_sched&ch=0&event=0

### 1.5.36 set.schedule.vca_email_sched

Example: set schedule vca email sched parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.vca_email_sched&ch=0&event=0&interval=20&&start_time[0][0]=00:00&end_time[0][0]=24:00&action_type[0][0]=8&start_time[0][1]=00:00&end_time[0][1]=12:00&action_type[0][1]=1&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=0&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

### 1.5.37 get.schedule.vca_others

| Parameter | Values | Description |
| --- | --- | --- |
| ch | integer | Channel id |
| triAlarms | integer | triAlarms:Alarm Input rec |
| triChannels | char | triChannels:Trigger Channel rec [char [64]] |
| event | integer | Smart event type 0:region entarnce 1:region exiting 2:advanced.motion detection 3:tamper detection 4:line crossing 5:loitering 6:human detection 7:people counting |

Example: set schedule var_alarm_output parametes

http://192.168.9.147/sdk.cgi?action=get.schedule.vca_others&ch=0&event=0

### 1.5.38 set.schedule.vca_others

Example: set schedule var_alarm_output parametes

http://192.168.9.147/sdk.cgi?action=set.schedule.vca_others&ch=0&event=0&triAlarms=63&triChannels=11111111111111111111111111111111

## 1.6 PLAYBACK

### 1.6.1 get.playback.pbdayinfo

| Parameter | Values | Description |
| --- | --- | --- |
| time | long | Start time available |
| chnid | integer | Channel ID |
| stream | integer | Stream type (0=main stream,1=sub stream) |
| dayinfo | String max 31 | A month information (no_record=0,exit_record =1) |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.playback.pbdayinfo&time=2016-03-07 00:00:00&chnid=1&stream=0

### 1.6.2 get.playback.recordinfo

| Parameter | Values | Description |
| --- | --- | --- |
| chnid | integer | Channel ID |
| stream | integer | Stream type (0=main stream,1=sub stream) |
| startTime | string max 31 | Start time string |
| endTime | string max 31 | End time string |
| cnt | integer max 100 | Record count |
| sid | interger | Play back Session ID |
| startTime | string max 31 | Start time string |
| endTime | string max 31 | end time string |
| recordType | integer | 0 : NONE 1<<0 : Timing 1<<1 : motion detect 1<<2 : alarm in 1<<3 : manual 1<<4 : anr 1<<5 : vca |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.playback.recordinfo&chnid=0&stream=0&startTime=2018-08-13 00:00:00&endTime=2018-08-13 23:59:59

### 1.6.3 get.playback.recordinfo_page

| Parameter | Values | Description |
| --- | --- | --- |
| sid | interger | Play back Session ID |
| page | interger | Page Number |
| cnt | interger max 100 | Record count |
| startTime | string max 31 | Start time string |
| endTime | string max 31 | End time string |
| recordType | integer | 0 : NONE 1<<0 : Timing 1<<1 : motion detect 1<<2 : alarm in 1<<3 : manual 1<<4 : anr 1<<5 : vca |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.playback.recordinfo_page&sid=0&page=2

### 1.6.4 get.playback.remote

| Parameter | Values | Description |
| --- | --- | --- |
| sid | interger | Session ID |
| real_time | String max 32 | Play time |
| except | interger | exception |

Example: get event parametes

http://192.168.9.129/sdk.cgi?action=get.playback.remote&chans=0&time=2016-03-07 12:00:00

### 1.6.5 get.playback.jump

| Parameter | Values | Description |
| --- | --- | --- |
| sessionId | integer | Session ID |
| jumpTime | String max 32 | Jump time string |

Example: get playback jump

http://192.168.9.129/sdk.cgi?action=get.playback.jump&sessionId=1&jumpTime=2016-03-06 06:44:33

### 1.6.6 get.playback.jump_batch

| Parameter | Values | Description |
| --- | --- | --- |
| cnt | integer | Session ID count |
| sid | integer | Session ID |
| jumpTime | String max 32 | Jump time string |

Example: get playback jump batch

http://192.168.9.129/sdk.cgi?action=get.playback.jump_batch&cnt=3&sid[0]=0&sid[1]=1&sid[2]=2&jumpTime=2018-08-12 05:23:25

### 1.6.7 get.playback.playtime

| Parameter | Values | Description |
| --- | --- | --- |
| sec | long | timestamp |
| curPlayTime | String max 32 |  |

Example: get playback playtime

http://192.168.9.129/sdk.cgi?action=get.playback.playtime&sid=1

### 1.6.8 get.playback.rtspurl

| Parameter | Values | Description |
| --- | --- | --- |
| sid | integer | Play back session ID |
| port | integer | RTSP Port |
| respath | String max 128 | RTSP resource path |

Example: get playback rtsp url

http://192.168.9.129/sdk.cgi?action=get.playback.rtspurl&sid=1

### 1.6.9 set.playback.pause

To set playback pause parametes

Example: set playback pause

http://192.168.9.129/sdk.cgi?action=set.playback.pause&sid=1

### 1.6.10 set.playback.pause_batch

| Parameter | Values | Description |
| --- | --- | --- |
| cnt | integer | Play back session count |
| sid | integer | Play back session ID |

Example: set playback pause batch

http://192.168.9.129/sdk.cgi?action=set.playback.pause_batch&cnt=3&sid[0]=0&sid[1]=1&sid[2]=2

### 1.6.11 set.playback.snapshot

| Parameter | Values | Description |
| --- | --- | --- |
| res | String max 32 | If channel n is successfully snapshot, res[n]=0,otherwise failed. |

Example: set playback snapshot

http://192.168.9.129/sdk.cgi?action=set.playback.snapshot&chnid=1

### 1.6.12 set.playback.snapshot_save

To set playback snapshot save parametes

Example: set playback snapshot save

http://192.168.9.129/sdk.cgi?action=set.playback.snapshot_save&disk=/media/sd1

### 1.6.13 set.playback.resume

To set playback resume parametes

Example: set playback resume

http://192.168.9.129/sdk.cgi?action=set.playback.resume&sid=1

### 1.6.14 set.playback.start

| Parameter | Values | Description |
| --- | --- | --- |
| chnid | integer | Channel ID |
| sid | integer | Play back Session ID |
| startTime | string | Start time string |
| endTime | string | End time string |
| playTime | string | Play time string |

Example: set playback start

http://192.168.9.129/sdk.cgi?action=set.playback.start&set.playback.start&chnid=2&sid=0&startTime=2018-08-16 00:00:00&endTime=2018-08-16 23:59:59&playTime=2018-08-16 10:13:40

### 1.6.15 set.playback.step

To set playback step parametes

Example: set playback step

http://192.168.9.129/sdk.cgi?action=set.playback.step&sid=1&direct=0

### 1.6.16 set.playback.step_batch

To set playback step parametes

Example: set playback step batch

http://192.168.9.129/sdk.cgi?action=set.playback.step_batch&direct=0&cnt=3&sid[0]=0&sid[1]=1&sid[2]=2

### 1.6.17 set.playback.stop

To set playback stop parametes

Example: set playback stop

http://192.168.9.129/sdk.cgi?action=set.playback.stop&sid=1

### 1.6.18 set.playback.stop_batch

To set playback stop parametes

Example: set playback stop batch

http://192.168.9.129/sdk.cgi?action=set.playback.stop_batch&cnt=3&sid[0]=0&sid[1]=1&sid[2]=2&close=0

### 1.6.19 set.playback.speed

To set playback speed parametes

Example: set playback speed

http://192.168.9.129/sdk.cgi?action=set.playback.speed&sid=1&direct=0&speed=2

### 1.6.20 set.playback.speed_batch

To set playback speed parametes

Example: set playback speed batch

http://192.168.9.129/sdk.cgi?action=set.playback.speed_batch&cnt=3&sid[0]=0&sid[1]=1&sid[2]=2&speed=1

### 1.6.21 set.playback.close

To set playback close parametes

Example: set playback close

http://192.168.9.129/sdk.cgi?action=set.playback.close&sid=1

### 1.6.22 set.playback.close_batch

To set playback close parametes

Example: set playback close batch

http://192.168.9.129/sdk.cgi?action=set.playback.close_batch&cnt=3&sid[0]=0&sid[1]=1&sid[2]=2

## 1.7 CAMERA

### 1.7.1 get.camera.ipcparam

| Parameter | Values |  |  |  | Description |
| --- | --- | --- | --- | --- | --- |
| ret | integer |  |  |  | 0 : success -1 : fail |
| chnid | integer |  |  |  | Channel id |
| struct ipc_param_range main_range | 24xres |  |  | width | width : integer height : integer framerate_max : integer framerate_min : integer bitrate_max : integer bitrate_min : integer vcodec_type: integer framerate : integer bitrate : integer |
|  |  |  |  | height |  |
|  | framerate_max |  |  |  |  |
|  | framerate_min |  |  |  |  |
|  | bitrate_max |  |  |  |  |
|  | bitrate_min |  |  |  |  |
|  | vcodec_type |  |  |  |  |
|  | cur_res |  | width |  |  |
|  |  |  | height |  |  |
|  | framerate |  |  |  |  |
|  | bitrate |  |  |  |  |
| struct ipc_param_range sub_range | 24xres |  | width |  | width : integer height : integer framerate_max : integer framerate_min : integer bitrate_max : integer bitrate_min : integer vcodec_type: integer framerate : integer bitrate : integer |
|  |  |  | height |  |  |
|  | framerate_max |  |  |  |  |
|  | framerate_min |  |  |  |  |
|  | bitrate_max |  |  |  |  |
|  | bitrate_min |  |  |  |  |
|  | Vcodec_type |  |  |  |  |
|  | cur_res | width |  |  |  |
|  |  | height |  |  |  |
|  | framerate |  |  |  |  |
|  | bitrate |  |  |  |  |
| sub_enable | Boolean |  |  |  | Enable sub streamer 0 : disable 1 : enable |
| audio_enable | Boolean |  |  |  | Enable audio 0 : disable 1 : enable |
| SmartStreamSupport | Boolean |  |  |  | Support smart stream 0 : nonsupport 1 : support |
| SmartStream | Boolean |  |  |  | Enable main smart stream 0 : disable 1 : enable |
| SmartStreamLevel | integer |  |  |  | Range 1~10 |
| SubSmartStream | Boolean |  |  |  | Enable sub smart stream 0 : disable 1 : enable |
| SubSmartStreamLevel | integer |  |  |  | Range 1~10 |

Example: get camera parametes

http://192.168.9.129/sdk.cgi?action=get.camera.ipcparam&chnid=1

### 1.7.2 set.camera.ipcparam

To set camera ipcparam parametes

Example: set camera ipcparam parametes

http://192.168.9.129/sdk.cgi?action=set.camera.ipcparam&chnid=0&mainFramerate=24&mainBitrate=4096&IframeInterval=20&subIframeInterval=50&subEnable=1&subFramerate=25&subBitrate=512&audioEnable=0&mainVideoType=0&videoTypeEnable=1&subVideoType=0&SmartStreamSupport=1&SmartStream=0&SmartStreamLevel=3&SubSmartStream=0&SubSmartStreamLevel=2&mainResWidth=1920&mainResHeight=1080&subResWidth=704&subResHeight=576

### 1.7.3 get.camera.ipclist

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| port | integer | IP Camera port |
| protocol | integer | IP Camera protocol |
| state | integer | IP Camera stte -2 : no_resource -1 : disconnect 0 : null 1 : waiting 2 : connect |
| main_rtsp_port | integer | Main RTSP port(554) |
| ipaddr | String max 32 | IP address |
| manufacturer | String max 64 | Manufacturer |
| hwid | String max 128 | Hardware ID |
| sn | String max 128 | Serial number |
| fwversion | String max 64 | Firmware version |
| mac | String max 48 | MAC |
| model | String max 64 | Model |
| model_id | integer | 0 : onvif 1 : rtsp 2 : canon |
| mrespath | String max 128 | Main streamer path |
| srespath | String max 128 | Sub streamer path |
| main_rtspurl | String max 128 | Main URL |
| sub_rtspurl | String max 128 | Sub URL |

Example: get camera parametes

http://192.168.9.129/sdk.cgi?action=get.camera.ipclist

### 1.7.4 add.camera.channel

To add camera channel parametes

Example: add camera channel parametes

http://192.168.9.129/sdk.cgi?action=add.camera.channel&id=1&name=CAM2&enable=1&ip_addr=192.168.9.126&manage_port=80&camera_protocol=0&name=CAM2&username=admin&password=ms1234&sub_rtsp_enable=1&sync_time=0&transmit_protocol=&poe_channel=&physical_port=&mac_addr=

### 1.7.5 modify.camera.channel

To modify camera channel parametes

Example: modify camera channel parametes

http://192.168.9.129/sdk.cgi?action=modify.camera.channel&id=1&name=CAM3&enable=1&ip_addr=192.168.9.126&manage_port=80&camera_protocol=0&name=CAM2&username=admin&password=ms1234&sub_rtsp_enable=1&sync_time=0&transmit_protocol=

### 1.7.6 remove.camera.channel

To remove camera channel parametes

Example: remove camera channel parametes

http://192.168.9.129/sdk.cgi?action=remove.camera.channel&chnid=1

### 1.7.7 get.camera.protocol

| Parameter | Values | Description |
| --- | --- | --- |
| pro_id | integer | Protocol id |
| pro_name | String max 64 | Protocol name |
| funtion | integer | Function value |
| enable | boolean | Enable protocol 0 : disable 1 : enable |
| display_model | boolean | Display 0 : dispaly 1 : nodisplay |

Example: get camera parametes

http://192.168.9.129/sdk.cgi?action=get.camera.protocol

### 1.7.8 search.camera.chns

| Parameter | Values | Description |
| --- | --- | --- |
| ipaddr | String max 32 | IP Camera address |
| mac | String max 32 | MAC |
| port | integer | Port |
| protocol | integer | Protocol 0 : onvif 1 : rtsp 2 : milesight 3 : canon 4 : psia |
| manufacturer | String max 64 | manufaturer |
| hwid | String max 128 | Hardware ID |
| sn | String max 128 | Serial number |
| fwversion | String max 64 | Firmware version |
| model | String max 64 | 0 : onvif 1 : rtsp 2 : canon |
| model_id | integer | Model ID |
| netmask | String max 32 | netmask |
| dns | String max 32 | dns |
| gateway | String max 32 | gateway |

Example: get camera parametes

http://192.168.9.129/sdk.cgi?action=search.camera.chns&pro_id=0

### 1.7.9 test.camera.ipconflict

To test camera ip conflict parametes

Example: test camera ip conflict parametes

http://192.168.9.129/sdk.cgi?action=test.camera.ipconflict&ip=192.168.9.127

### 1.7.10 test.camera.ipcconn

To test camera ip conflict parametes

Example: test camera ipc connenct

http://192.168.9.129/sdk.cgi?action=test.camera.ipcconn&ip=192.168.9.126&port=80&user=admin&password=1111&protocol=0&model=0

### 1.7.11 get.camera.rtsp

To get camera irtsp info parametes

Example: get camera ipc connenct

http://192.168.9.129/sdk.cgi?action=get.camera.rtsp

### 1.7.12 get.camera.imageparam

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel id |
| brightness | integer |  |
| contrast | integer |  |
| saturation | integer |  |
| result | integer | result |

Example: get camera imageparam

http://192.168.9.129/sdk.cgi?action=get.camera.imageparam&chnid=0

### 1.7.13 set.camera.imageparam

To set camera imageparam info parametes

Example: set camera imageparam

http://192.168.9.129/sdk.cgi?action=set.camera.imageparam&chnid=0&brightness=127&contrast=127&saturation=127

### 1.7.12 get.camera.imageparam

| Parameter |  | Values | Description |
| --- | --- | --- | --- |
| ch |  | integer | Channel id |
| 4xPrivacy_mask | area_id | struct | area_id: privacy_mask area id [integer] |
|  | enable |  | enable: [integer] |
|  | start_x |  | start_x:set start x [integer] |
|  | start_y |  | start_y:set start y [integer] |
|  | area_width |  | area_width:privacy_mask area width[integer] |
|  | area_height |  | area_height:privacy_mask area height [integer] |
|  | screen_width |  | screen_width:[integer] 480 |
|  | screen_height |  | screen_height:[integer] 270 |
|  | fill_color |  | fill_color:privacy_mask color [integer] |

### 1.7.14 get.camera.privacy_mask

Example: get camera privacy_mask

http://192.168.9.147/sdk.cgi?action=get.camera.privacy_mask&chnid=0

### 1.7.13 set.camera.imageparam

To set camera iprivacy_mask parametes

### 1.7.15 set.camera.privacy_mask

Example: set camera privacy_mask

http://192.168.9.147/sdk.cgi?action=set.camera.privacy_mask&ch=0&area_id[0]=0&enable[0]=1&start_x[0]=55&start_y[0]=44&area_width[0]=91&area_height[0]=60&screen_width[0]=480&screen_height[0]=270&fill_color[0]=0&area_id[1]=1&enable[1]=1&start_x[1]=213&start_y[1]=60&area_width[1]=105&area_height[1]=80&screen_width[1]=480&screen_height[1]=270&fill_color[1]=0&area_id[2]=2&enable[2]=1&start_x[2]=74&start_y[2]=155&area_width[2]=76&area_height[2]=56&screen_width[2]=480&screen_height[2]=270&fill_color[2]=0&area_id[3]=3&enable[3]=1&start_x[3]=252&start_y[3]=174&area_width[3]=120&area_height[3]=80&screen_width[3]=480&screen_height[3]=270&fill_color[3]=0

| Parameter | Values | Description |
| --- | --- | --- |
| ip | String max 32 |  |
| port | integer |  |
| user | String max 64 |  |
| password | String max 32 |  |
| ret | integer | result |

Example: try camera connectoin

http://192.168.9.147/sdk.cgi?action=test.try.camera.ipcconn&ip=192.168.10.234&port=80&user=admin&password=1234&protocol=0

| Parameter | Values | Description |
| --- | --- | --- |
| chnid | integer | Channel id |
| smartir | integer | Smart IR |
| whiteblance | integer | White Balance |
| focusmode | integer |  |
| antiblurry | integer |  |
| wdrenable | integer | Wide Dynamic Range |
| wdrlevel | integer | Wide Dynamic Level |
| wdrstarthour | integer | Start Time |
| wdrstartminute | integer | Start Time |
| wdrstophour | integer | End Time |
| wdrstopminute | integer | End Time |
| regiontype | integer |  |
| hlcmode | integer | High Light Compensation |
| exclude | integer |  |
| exposemode | integer |  |
| modelType | integer |  |
| exposeregion | String max 64 |  |

Example: get camera enhance

http://192.168.9.147/sdk.cgi?action=get.camera.ipcimage.enhance&chnid=0

### 1.7.13 set.camera.imageparam

To set camera enhance parametes

### 1.7.15 set.camera.privacy_mask

Example: set camera enhance

http://192.168.9.150/sdk.cgi?action=set.camera.ipcimage.enhance&chnid=0&smartir=0&whiteblance=0&wdrenable=0&wdrlevel=2&wdrstarthour=6&wdrstartminute=0&wdrstophour=18&wdrstopminute=0&hlcmode=0

| Parameter | Values | Description |
| --- | --- | --- |
| chnid | integer | Channel id |
| colorkiller | integer | Day/Night Mode |
| modestarthour | integer | Start time of Nigh |
| modestartminute | integer | Start time of Nigh |
| modestophour | integer | End time of Night |
| modestopminute | integer | End time of Night |
| exposurectrl | integer | Power Line Frequency |
| lighttype | integer |  |
| mirctrl | integer | Video Orientation |
| localdisplay | integer |  |
| ircutaddaytonight | integer |  |
| ircutadnighttoday | integer |  |
| corridormode | integer | Corridor Mode |
| imagerotation | integer | Image Rotation |
| saveCorridor | integer | If saveCorridor=0 save mirctrl, If saveCorridor=1 save corridormode & imagerotation |

Example: get camera display

http://192.168.9.147/sdk.cgi?action=get.camera.ipcimage.display&chnid=0

### 1.7.13 set.camera.imageparam

To set camera display parametes

### 1.7.14 set.camera.privacy_mask

Example: set camera display

http://192.168.9.147/sdk.cgi?action=set.camera.ipcimage.display&chnid=0&colorkiller=2&modestarthour=18&modestartminute=0&modestophour=6&modestopminute=0&exposurectrl=1&mirctrl=-1&ircutaddaytonight=303&ircutadnighttoday=451&corridormode=0&imagerotation=0&saveCorridor=0

| Parameter | Values | Description |
| --- | --- | --- |
| ret_cnt | integer | Channel num |
| Chanid(array) | integer | Channel id |
| model(array) | integer | Channel model |
| fwversion(array) | integer | Channel firmware version |

Example: get camera model info

http://192.168.9.147/sdk.cgi?action=get.camera.all.modelinfo

To set camera fisheye mode

Example: set camera fisheye mode

http://192.168.9.129/sdk.cgi?action=set.camera.fisheye.mode&chnid=1&fishmount=1&fishdisplay=4

| Parameter | Values | Description |
| --- | --- | --- |
| chnid | integer | Channel id |
| fishmount | integer | Fishmount mode |
| fishdisplay | integer | displaymode |

### 1.7.15 get.camera.privacy_mask

Example: get camera fisheye mode

http://192.168.9.147/sdk.cgi?action=get.camera.fisheye.mode&chnid=1

### 1.7.16 get.camera.ipctype

| Parameter | Values | Description |
| --- | --- | --- |
| chnid | integer | Channel id |

Example: get camera type

http://192.168.9.129/sdk.cgi?action=get.camera.ipctype&chnid=0

### 1.7.17 get.camera.osd

| Parameter | Values | Description |
| --- | --- | --- |
| chnid | integer | Channel id |
| type | integer | 0:All streams 1:Primary stream 2:Secondary stream |
| enable_text | boolean | Enable show main video title 0 : disable 1 : enable |
| text_name | String max 32 | Main video title |
| textpos | integer | select main title position 0:Top-Left 1:Top-Right 2:Bottom-Left 3:Bottom-Right |
| enable_date | integer | Enable show main date 0 : disable 1 : enable |
| datepos | integer | select main date position 0:Top-Left 1:Top-Right 2:Bottom-Left 3:Bottom-Right |
| dateformat | integer | select main date format 0:YYYY-MM-DD 1:MM-DD-YYYY 2:DD-MM-YYYY |
| enable_text_sub | boolean | Enable show sub video title 0 : disable 1 : enable |
| text_name_sub | String max 32 | Sub video title |
| textpos_sub | integer | select subtitle position 0:Top-Left 1:Top-Right 2:Bottom-Left 3:Bottom-Right |
| enable_date_sub | integer | Enable show sub date 0 : disable 1 : enable |
| datepos_sub | integer | select sub date position 0:Top-Left 1:Top-Right 2:Bottom-Left 3:Bottom-Right |
| dateformat_sub |  | select sub date format 0:YYYY-MM-DD 1:MM-DD-YYYY 2:DD-MM-YYYY |

Example: get osd info

http://192.168.9.129/sdk.cgi?action=get.camera.osd&chnid=0

### 1.7.18 set.camera.osd

To set camera osd info

Example: set camera osd info

http://192.168.2.134/sdk.cgi?action=set.camera.osd&type=1&chnid=0&enable_text=1&text_name=main_test&textpos=0&enable_date=1&datepos=1&dateformat=2&enable_text_sub=1&text_name_sub=sub test&textpos_sub=0&enable_date_sub=1&datepos_sub=1&dateformat_sub=2

### 1.7.19 get.camera.vca.region.entarnce

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| enable | integer | 0:disable 1:enable |
| region | String 300 | Region area 0:disable 1:enable |

Example: get vca region entarnce info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.region.entarnce&chnid=0

### 1.7.20 set.camera.vca.region.entarnce

Example: set vca region entarnce info

http://192.168.9.129/sdk.cgi?action=set.camera.vca.region.entarnce&chnid=0&enable=0&region=11111000000000000000000000000000000000000000000000011111111111000000000000111111111110000000000001111111111100000000000011111111111000000001111000111111110000000011110001111111100000000111111100000000000000001111111000000000000000011111110000000000000000000000000000000000000000000000000000000000000

### 1.7.21 get.camera.vca.region.exiting

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| enable | integer | 0:disable 1:enable |
| region | String 300 | Region area 0:disable 1:enable |

Example: get vca region exiting info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.region.exiting&chnid=0

### 1.7.22 set.camera.vca.region.exiting

Example: set vca region exiting info

http://192.168.9.129/sdk.cgi?action=set.camera.vca.region.exiting&chnid=0&enable=0&region=11111000000000000000000000000000000000000000000000011111111111000000000000111111111110000000000001111111111100000000000011111111111000000001111000111111110000000011110001111111100000000111111100000000000000001111111000000000000000011111110000000000000000000000000000000000000000000000000000000000000

### 1.7.23 get.camera.vca.advanced.motion

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| enable | integer | 0:disable 1:enable |
| sensitivity | integer | Sensitivity:1-10 |
| region | String 300 | Region area 0:disable 1:enable |

Example: get vca advanced motion info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.advanced.motion&chnid=0

### 1.7.24 set.camera.vca.advanced.motion

Example: set vca advanced motion info

http://192.168.9.129/sdk.cgi?action=set.camera.vca.advanced.motion&chnid=0&enable=1&sensitivity=9&region=11111000000000000000000000000000000000000000000000011111111111000000000000111111111110000000000001111111111100000000000011111111111000000001111000111111110000000011110001111111100000000111111100000000000000001111111000000000000000011111110000000000000000000000000000000000000000000000000000000000000

### 1.7.25 get.camera.vca.tamper.detection

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| enable | integer | 0:disable 1:enable |
| sensitivity | integer | Sensitivity:1-10 |

Example: get vca tamper detection info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.tamper.detection&chnid=0

### 1.7.26 set.camera.vca.tamper.detection

Example: set vca tamper detection info

http://192.168.9.129/sdk.cgi?action=set.camera.vca.tamper.detection&chnid=0&enable=1&sensitivity=9

### 1.7.27 get.camera.vca.line.crossing

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| enable | integer | 0:disable 1:enable |
| width | integer | Width |
| height | integer | Height |
| startX | integer | X of start point |
| StartY | integer | Y of start point |
| stopX | integer | X of stop point |
| stopY | integer | Y of stop point |
| direction | integer | Direction of line 0:A->B 1:B->A 2:A<->B |

Example: get vca line crossing info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.line.crossing&chnid=0

### 1.7.28 set.camera.vca.line.crossing

Example: set vca line crossing info

http://192.168.9.129/sdk.cgi?action=set.camera.vca.line.crossing&chnid=0&enable=1&startX0=1518&startY0=972&stopX0=1482&stopY0=121&direction0=0&startX1=0&startY1=0&stopX1=0&stopY1=0&direction1=0&startX2=0&startY2=0&stopX2=0&stopY2=0&direction2=0&startX3=10&startY3=10&stopX3=1&stopY3=1&direction3=3

### 1.7.29 get.camera.vca.loitering

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| enable | integer | 0:disable 1:enable |
| minLoiteringTime | integer | Min loitering time(3-300s) |
| loiteringObjectSize | integer | Object size (1-100) |
| region | String 300 | Region area 0:disable 1:enable |

Example: get vca loitering info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.loitering&chnid=0

### 1.7.30 set.camera.vca.loitering

Example: set vca loitering info

http://192.168.9.129/sdk.cgi?action=set.camera.vca.loitering&chnid=0&enable=1&minLoiteringTime=30&loiteringObjectSize=5&region=11111000000000000000000000000000000000000000000000011111111111000000000000111111111110000000000001111111111100000000000011111111111000000001111000111111110000000011110001111111100000000111111100000000000000001111111000000000000000011111110000000000000000000000000000000000000000000000000000000000000

### 1.7.31 get.camera.vca.human.detection

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| enable | integer | 0:disable 1:enable |
| tracks | integer | Tracks 0:disable 1:enable |

Example: get vca human detection info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.human.detection&chnid=0

### 1.7.32 set.camera.vca.human.detection

Example: set vca human detection info

http://192.168.9.129/sdk.cgi?action=set.camera.vca.human.detection&chnid=0&enable=1&tracks=1

### 1.7.33 get.camera.vca.people.counting

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| enable | integer | 0:disable 1:enable |
| width | integer | Width |
| height | integer | Height |
| startX | integer | X of start point |
| StartY | integer | Y of start point |
| stopX | integer | X of stop point |
| stopY | integer | Y of stop point |
| osdEnable | integer | Show counting OSD 0:disable 1:enable |
| osdTextPosition | integer | Text position 0:Top-Left 1:Top-Right 2:Bottom-Left 3:Bottom-Right |
| fontSize | integer | Font Size 0:Smallest 1:Small 2:Medium 3:Large 4:Largest 5:Auto |
| triggerEnable | integer | Enable Alarm 0:disable 1:enable |
| triggerEnableByIn | integer |  |
| inThresholds | integer |  |
| triggerEnableByOut | integer |  |
| outThresholds | integer |  |
| triggerEnableBySum | integer |  |
| sumThresholds | integer |  |

Example: get vca people counting info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.people.counting&chnid=0

### 1.7.34 set.camera.vca.people.counting

Example: set vca people counting info

http://192.168.9.129/sdk.cgi?action=set.camera.vca.people.counting&chnid=0&enable=1&startX=10&startY=10&stopX=300&stopY=100&osdEnable=1&osdTextPosition=0&fontSize=0&triggerEnable=0&triggerEnableByIn=0&inThresholds=1&triggerEnableByOut=0&outThresholds=1&triggerEnableBySum=0&sumThresholds=1

### 1.7.35 get.camera.vca.settings

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| enable | integer | 0:disable 1:enable |
| width | integer | Width |
| height | integer | Height |
| processFps | integer | Process FPS |
| scenarioMode | integer | Scenario Mode 0:Outdoor 1:Indoor |
| cameraInstallation | integer | Camera Installation 0:Horizontal View 1:Overhead View 2:Angle View |
| analysisType | integer | Analysis Type 2:Standard 3:Advanced |
| minObjectX | integer | Start point x of min object |
| minObjectY | integer | Start point y of min object |
| minObjectWidth | integer | Width of min object |
| minObjectHeight | integer | Height of min object |
| maxObjectX | integer | Start point x of max object |
| maxObjectY | integer | Start point y of max object |
| maxObjectWidth | integer | Width of max object |
| maxObjectHeight | integer | Height of max object |

Example: get vca settings info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.settings&chnid=0

### 1.7.36 set.camera.vca.settings

Example: set vca settings info

http://192.168.9.129/sdk.cgi?action=set.camera.vca.settings&processFps=5&scenarioMode=0&cameraInstallation=2&analysisType=2&minObjectX=10&minObjectY=10&minObjectWidth=100&minObjectHeight=100&maxObjectX=50&maxObjectY=50&maxObjectWidth=200&maxObjectHeight=200

### 1.7.37 get.camera.vca.lisence

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| licenseStatus | integer | License status 0:Invalid 1:valid |
| license | String | License |

Example: get vca lisence info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.lisence&chnid=0

### 1.7.38 set.camera.vca.lisence

Example: set vca lisence info

http://192.168.9.129/sdk.cgi?action=set.camera.vca.lisence&license=yzyJ32xhuECumunnFFGGHHhhiiyI

### 1.7.39 set.camera.vca.clearcount

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |

Example: set vca clearcount info

http://192.168.9.129/sdk.cgi?action=set.camera.vca.clearcount&chnid=0

### 1.7.40 get.camera.vca.support

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| support | integer | Support 0:no 1:yes |

Example: get vca support info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.support&chnid=0

### 1.7.41 get.camera.vca.support_all

| Parameter | Values | Description |
| --- | --- | --- |
| support | string | Support 0:no 1:yes |

Example: get vca support all info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.support_all

### 1.7.42 get.camera.vca.person_point

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| enable | integer | 0:disable 1:enable |
| width | integer | Width |
| height | integer | Height |
| person_point_cnt | integer | Point count |
| id | integer | Point id |
| type | integer | Point type |
| startX | integer | Start point x |
| startY | integer | Start point y |
| stopX | integer | Stop point x |
| stopY | integer | Stop point y |

Example: get vca person point info

http://192.168.9.129/sdk.cgi?action=get.camera.vca.person_point&chnid=0

## 1.8 STATUS.

### 1.8.1 get.status.allipc

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| port | integer | IP Camera port |
| protocol | integer | IP Camera protocol |
| state | integer | IP Camera stte -2 : no_resource -1 : disconnect 0 : null 1 : waiting 2 : connect |
| main_rtsp_port | integer | Main RTSP port(554) |
| ipaddr | String max 32 | IP address |
| manufacturer | String max 64 | Manufacturer |
| hwid | String max 128 | Hardware ID |
| sn | String max 128 | Serial number |
| fwversion | String max 64 | Firmware version |
| mac | String max 48 | MAC |
| model | String max 64 | Model |
| model_id | integer | 0 : onvif 1 : rtsp 2 : canon |
| mrespath | String max 128 | Main streamer path |
| srespath | String max 128 | Sub streamer path |
| main_rtspurl | String max 128 | Main URL |
| sub_rtspurl | String max 128 | Sub URL |

Example: get status parametes

http://192.168.9.129/sdk.cgi?action=get.status.allipc

### 1.8.2 get.status.ipcinfo

| Parameter | Values | Description |
| --- | --- | --- |
| chanid | integer | Channel ID |
| port | integer | IP Camera port |
| protocol | integer | IP Camera protocol |
| state | integer | IP Camera stte -2 : no_resource -1 : disconnect 0 : null 1 : waiting 2 : connect |
| main_rtsp_port | integer | Main RTSP port(554) |
| ipaddr | String max 32 | IP address |
| manufacturer | String max 64 | Manufacturer |
| hwid | String max 128 | Hardware ID |
| sn | String max 128 | Serial number |
| fwversion | String max 64 | Firmware version |
| mac | String max 48 | MAC |
| model | String max 64 | Model |
| model_id | integer | 0 : onvif 1 : rtsp 2 : canon |
| mrespath | String max 128 | Main streamer path |
| srespath | String max 128 | Sub streamer path |
| main_rtspurl | String max 128 | Main URL |
| sub_rtspurl | String max 128 | Sub URL |

Example: get status parametes

http://192.168.9.129/sdk.cgi?action=get.status.ipcinfo&ch=0

### 1.8.3 get.status.ipcstatus

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| main | cur_res | width | integer |  |
|  |  | height | integer |  |
|  | frame_rate |  | integer |  |
|  | bit_rate |  | integer |  |
|  | connection |  | integer |  |
| sub | cur_res | width | integer |  |
|  |  | height | integer |  |
|  | frame_rate |  | integer |  |
|  | bit_rate |  | integer |  |
|  | connection |  | integer |  |
| record |  |  | integer |  |
| alarm |  |  | integer |  |
| chnid |  |  | integer |  |

Example: get status parametes

http://192.168.9.129/sdk.cgi?action=get.status.ipcstatus

### 1.8.4 get.status.event

| Parameter | Values | Description |
| --- | --- | --- |
| video_loss | Long long | Bit mask, each bit represent a channel |
| motion_detect | Long long | Similar as above |
| alarm_out | integer | Alarm out bit mask |
| alarm_in | sensor | Sensor bit mask |
| regionin_detect | Long long | Bit mask, each bit represent a channel |
| regionexit_detect | Long long | Bit mask, each bit represent a channel |
| advanced_motion_detect | Long long | Bit mask, each bit represent a channel |
| tamper_detect | Long long | Bit mask, each bit represent a channel |
| linecross_detect | Long long | Bit mask, each bit represent a channel |
| loiter_detect | Long long | Bit mask, each bit represent a channel |
| human_detect | Long long | Bit mask, each bit represent a channel |
| in | integer | People count of in |
| out | integer | People count of out |

Example: get status parametes

http://192.168.9.129/sdk.cgi?action=get.status.event

### 1.8.5 get.status.ioalarm

| Parameter | Values | Description |
| --- | --- | --- |
| alarm | integer | Result of io alarm status |

Example: get status parametes

http://192.168.9.129/sdk.cgi?action=get.status.ioalarm

### 1.8.6 get.status.iosensor

| Parameter | Values | Description |
| --- | --- | --- |
| sensor | integer | Result of io sensor status |

Example: get status parametes

http://192.168.9.129/sdk.cgi?action=get.status.iosensor

### 1.8.7 get.status.netbandwidth

| Parameter | Values | Description |
| --- | --- | --- |
| total | integer | Total bandwidth |
| free | integer | Free bandwidth |
| used | integer | Used bandwidth |

Example: get status parametes

http://192.168.9.129/sdk.cgi?action=get.status.netbandwidth

### 1.8.8 get.status.netinfo

| Parameter | Values | Description |
| --- | --- | --- |
| ifname | String max 10 | Lan1/Lan2/Bond0 |
| netif_id | integer | Network card ID |
| conn_stat | integer | Connected Saturation 0 : unconnected 1 : connected |
| speed_mode | integer | Speed mode 10M/s 100M/s |
| duplex | integer | Duplex 0 : Half-duplex 1 : fuul-duplex |
| enable | Boolean | Enable 0 : disable 1 : enable |
| dhcp_enable | Boolean | Enable 0 : disable 1 : enable |
| ip | String max 24 | IP address |
| mac | String max 24 | MAC |
| primary_dns | String max 24 | Preferred DNS |
| second_dns | String max 24 | Alternate DNS |
| gateway | String max 24 | Gateway |
| netmask | String max 24 | Net mask |
| mtu | integer | MTU(B) |
| ret_cnt | integer | Get status netinfo result count |

Example: get status parametes

http://192.168.9.129/sdk.cgi?action=get.status.netinfo

### 1.8.9 get.status.netspeed

| Parameter | Values | Description |
| --- | --- | --- |
| ifname | String max 10 | Network Card Name LAN1/LAN2/BOND0 |
| rx_speed | integer | Receive Rate(Unit:Bps) |
| tx_speed | integer | Send Rate(Unit: Bps) |
| netif_id | integer | Network Card ID |

Example: get status parametes

http://192.168.9.129/sdk.cgi?action=get.status.netspeed

Example: get motion status parametes

http://192.168.9.129/sdk.cgi?action=get.status.motion

## 1.9 RECORD

### 1.9.1 get.record.advanced

| Parameter | Values | Description |
| --- | --- | --- |
| ch | Integer 0~15 | Channel id |
| mode | integer | Record mode 0 : Never 1 : Always 2 : By Schedule |
| audio_on | boolean | Enable audio 0 : disable 1 : enable |
| prev_rec_on | boolean | Enable pre-record 0 : disable 1 : enable |
| prev_rec_duration | Integer 0~10s | Pre-record time(s) |
| post_rec_on | boolean | Enable post-record 0 : disable 1 : enable |
| post_rec_duration | Integer 1~60 min | Post-record time(min) |
| rec_expiration_date | Integer 0~60 day | Expiration date record (day) |
| photo_expiration_date | Integer | Photo expiration time(day) 0:unlimit other:days |

Example: get record parametes

http://192.168.9.129/sdk.cgi?action=get.record.advanced&id=0

### 1.9.2 set.record.advanced

To set record advanced parametes

Example: set record advanced

http://192.168.9.129/sdk.cgi?action=set.record.advanced&id=0&mode=2&audio_enable=0&prev_record_on=1&prev_record_duration=10&post_record_on=1&post_record_duration=3&record_expiration_date=0

### 1.9.3 set.record.record

To set record parametes

Example: set record record parametes

http://192.168.9.129/sdk.cgi?action=set.record.record&chnid=0&enable=0&event=1

### 1.9.4 set.record.mode

To set record mode parametes

Example: set record mode

http://192.168.9.129/sdk.cgi?action=set.record.mode&id=0&mode=0

### 1.9.5 get.record.schedule

| Parameter |  |  | Values | Description |
| --- | --- | --- | --- | --- |
| 8xschedule_day | 12xschedule_item | start_time | struct | start_time : start rec time [char [32]] end_time : end rec time [char [32]] action_type : rec type [integer] Wholeday_enable : 0/1 [integer] Wholeday_action_type : wholeday action type [integer] |
|  |  | end_time |  |  |
|  |  | action_type |  |  |
|  | wholeday_enable |  |  |  |
|  | wholeday_action_type |  |  |  |
| enable |  |  | integer | ~ |
| ch |  |  | integer | Channel id |

Example: get record schedule

http://192.168.9.129/sdk.cgi?action=get.record.schedule&ch=0

### 1.9.6 set.record.schedule

To set schedule recsched parametes

Example: set schedule recsched parametes

http://192.168.9.129/sdk.cgi?action=set.record.schedule&ch=0&enable=1&wholeday_enable[0]=1&wholeday_action_type[0]=1&start_time[0][0]=00:00&end_time[0][0]=01:00&action_type[0][0]=0&start_time[0][1]=00:00&end_time[0][1]=12:00&action_type[0][1]=1&start_time[0][2]=&end_time[0][2]=&action_type[0][2]=0&start_time[0][3]=&end_time[0][3]=&action_type[0][3]=0&start_time[0][4]=&end_time[0][4]=&action_type[0][4]=0&start_time[0][5]=&end_time[0][5]=&action_type[0][5]=0&start_time[0][6]=&end_time[0][6]=&action_type[0][6]=0&start_time[0][7]=&end_time[0][7]=&action_type[0][7]=0&start_time[0][8]=&end_time[0][8]=&action_type[0][8]=0&start_time[0][9]=&end_time[0][9]=&action_type[0][9]=0&start_time[0][10]=&end_time[0][10]=&action_type[0][10]=0&start_time[0][11]=12:00&end_time[0][11]=14:00&action_type[0][11]=2&wholeday_enable[1]=0&wholeday_action_type[1]=0&start_time[1][0]=&end_time[1][0]=&action_type[1][0]=0&start_time[1][1]=&end_time[1][1]=&action_type[1][1]=0&start_time[1][2]=&end_time[1][2]=&action_type[1][2]=0&start_time[1][3]=&end_time[1][3]=&action_type[1][3]=0&start_time[1][4]=&end_time[1][4]=&action_type[1][4]=0&start_time[1][5]=&end_time[1][5]=&action_type[1][5]=0&start_time[1][6]=&end_time[1][6]=&action_type[1][6]=0&start_time[1][7]=&end_time[1][7]=&action_type[1][7]=0&start_time[1][8]=&end_time[1][8]=&action_type[1][8]=0&start_time[1][9]=&end_time[1][9]=&action_type[1][9]=0&start_time[1][10]=&end_time[1][10]=&action_type[1][10]=0&start_time[1][11]=&end_time[1][11]=&action_type[1][11]=0&wholeday_enable[2]=0&wholeday_action_type[2]=0&start_time[2][0]=&end_time[2][0]=&action_type[2][0]=0&start_time[2][1]=&end_time[2][1]=&action_type[2][1]=0&start_time[2][2]=&end_time[2][2]=&action_type[2][2]=0&start_time[2][3]=&end_time[2][3]=&action_type[2][3]=0&start_time[2][4]=&end_time[2][4]=&action_type[2][4]=0&start_time[2][5]=&end_time[2][5]=&action_type[2][5]=0&start_time[2][6]=&end_time[2][6]=&action_type[2][6]=0&start_time[2][7]=&end_time[2][7]=&action_type[2][7]=0&start_time[2][8]=&end_time[2][8]=&action_type[2][8]=0&start_time[2][9]=&end_time[2][9]=&action_type[2][9]=0&start_time[2][10]=&end_time[2][10]=&action_type[2][10]=0&start_time[2][11]=&end_time[2][11]=&action_type[2][11]=0&wholeday_enable[3]=0&wholeday_action_type[3]=0&start_time[3][0]=&end_time[3][0]=&action_type[3][0]=0&start_time[3][1]=&end_time[3][1]=&action_type[3][1]=0&start_time[3][2]=&end_time[3][2]=&action_type[3][2]=0&start_time[3][3]=&end_time[3][3]=&action_type[3][3]=0&start_time[3][4]=&end_time[3][4]=&action_type[3][4]=0&start_time[3][5]=&end_time[3][5]=&action_type[3][5]=0&start_time[3][6]=&end_time[3][6]=&action_type[3][6]=0&start_time[3][7]=&end_time[3][7]=&action_type[3][7]=0&start_time[3][8]=&end_time[3][8]=&action_type[3][8]=0&start_time[3][9]=&end_time[3][9]=&action_type[3][9]=0&start_time[3][10]=&end_time[3][10]=&action_type[3][10]=0&start_time[3][11]=&end_time[3][11]=&action_type[3][11]=0&wholeday_enable[4]=0&wholeday_action_type[4]=0&start_time[4][0]=&end_time[4][0]=&action_type[4][0]=0&start_time[4][1]=&end_time[4][1]=&action_type[4][1]=0&start_time[4][2]=&end_time[4][2]=&action_type[4][2]=0&start_time[4][3]=&end_time[4][3]=&action_type[4][3]=0&start_time[4][4]=&end_time[4][4]=&action_type[4][4]=0&start_time[4][5]=&end_time[4][5]=&action_type[4][5]=0&start_time[4][6]=&end_time[4][6]=&action_type[4][6]=0&start_time[4][7]=&end_time[4][7]=&action_type[4][7]=0&start_time[4][8]=&end_time[4][8]=&action_type[4][8]=0&start_time[4][9]=&end_time[4][9]=&action_type[4][9]=0&start_time[4][10]=&end_time[4][10]=&action_type[4][10]=0&start_time[4][11]=&end_time[4][11]=&action_type[4][11]=0&wholeday_enable[5]=0&wholeday_action_type[5]=0&start_time[5][0]=&end_time[5][0]=&action_type[5][0]=0&start_time[5][1]=&end_time[5][1]=&action_type[5][1]=0&start_time[5][2]=&end_time[5][2]=&action_type[5][2]=0&start_time[5][3]=&end_time[5][3]=&action_type[5][3]=0&start_time[5][4]=&end_time[5][4]=&action_type[5][4]=0&start_time[5][5]=&end_time[5][5]=&action_type[5][5]=0&start_time[5][6]=&end_time[5][6]=&action_type[5][6]=0&start_time[5][7]=&end_time[5][7]=&action_type[5][7]=0&start_time[5][8]=&end_time[5][8]=&action_type[5][8]=0&start_time[5][9]=&end_time[5][9]=&action_type[5][9]=0&start_time[5][10]=&end_time[5][10]=&action_type[5][10]=0&start_time[5][11]=&end_time[5][11]=&action_type[5][11]=0&wholeday_enable[6]=0&wholeday_action_type[6]=0&start_time[6][0]=&end_time[6][0]=&action_type[6][0]=0&start_time[6][1]=&end_time[6][1]=&action_type[6][1]=0&start_time[6][2]=&end_time[6][2]=&action_type[6][2]=0&start_time[6][3]=&end_time[6][3]=&action_type[6][3]=0&start_time[6][4]=&end_time[6][4]=&action_type[6][4]=0&start_time[6][5]=&end_time[6][5]=&action_type[6][5]=0&start_time[6][6]=&end_time[6][6]=&action_type[6][6]=0&start_time[6][7]=&end_time[6][7]=&action_type[6][7]=0&start_time[6][8]=&end_time[6][8]=&action_type[6][8]=0&start_time[6][9]=&end_time[6][9]=&action_type[6][9]=0&start_time[6][10]=&end_time[6][10]=&action_type[6][10]=0&start_time[6][11]=&end_time[6][11]=&action_type[6][11]=0&wholeday_enable[7]=0&wholeday_action_type[7]=0&start_time[7][0]=&end_time[7][0]=&action_type[7][0]=0&start_time[7][1]=&end_time[7][1]=&action_type[7][1]=0&start_time[7][2]=&end_time[7][2]=&action_type[7][2]=0&start_time[7][3]=&end_time[7][3]=&action_type[7][3]=0&start_time[7][4]=&end_time[7][4]=&action_type[7][4]=0&start_time[7][5]=&end_time[7][5]=&action_type[7][5]=0&start_time[7][6]=&end_time[7][6]=&action_type[7][6]=0&start_time[7][7]=&end_time[7][7]=&action_type[7][7]=0&start_time[7][8]=&end_time[7][8]=&action_type[7][8]=0&start_time[7][9]=&end_time[7][9]=&action_type[7][9]=0&start_time[7][10]=&end_time[7][10]=&action_type[7][10]=0&start_time[7][11]=&end_time[7][11]=&action_type[7][11]=0

| Parameter | Values | Description |
| --- | --- | --- |
| record | char | Batch setting reord start [char[64]] |
| stop | char | Batch setting reord stop [char[64]] |

### 1.9.7 set.record.batchsetting

Example: set camera batchsetting

http://192.168.9.147/sdk.cgi?action=set.record.batchsetting&record=10101010101010101010101010101010&stop=01010101010101010101010101010101

## 1.10 PTZ

### 1.10.1 set.ptz.auto_focus

To set ptz auto focus parametes

Example: set ptz auto focus parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.auto_focus&chanid=

### 1.10.2 set.ptz.auto_scan

To set ptz auto scan parametes

Example: set ptz auto scan parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.auto_scan&chanid=

### 1.10.3 set.ptz.brush_off

To set ptz brush off parametes

Example: set ptz brush off parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.brush_off&chanid=

### 1.10.4 set.ptz.brush_on

To set ptz brush on parametes

Example: set ptz brush on parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.brush_on&chanid=

### 1.10.5 set.ptz.down

To set ptz down parametes

Example: set ptz down parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.down&chanid=&speed=

### 1.10.5 set.ptz.focus_in

To set ptz focus in parametes

Example: set ptz focus in parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.focus_in&chanid=&speed=

### 1.10.6 set.ptz.focus_out

To set ptz focus out parametes

Example: set ptz focus out parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.focus_out&chanid=&speed=

### 1.10.7 set.ptz.focus_stop

To set ptz focus stop parametes

Example: set ptz focus stop parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.focus_stop&chanid=

### 1.10.8 get.ptz.serial_port

| Parameter | Values | Description |
| --- | --- | --- |
| id | integer | Channel id |
| baudrate | boolean | PTZ baudrate |
| data_bit | An unsigned integer | Data bit |
| stop_bit | An unsigned integer | Stop bid |
| parity_type | integer | Parity type 0 : none 1 : Even 2 : Odd |
| protocol | integer | PTZ Protocol |
| address | Integer 1~255 | PTZ address |
| com_type | integer |  |
| connect_type | integer | Connect type 0 : onvif 1 : RS485 |

### 1.10.9 set.ptz.iris_minus

To set ptz iris minus parametes

Example: set ptz iris minus  parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.iris_minus&chanid=

### 1.10.10 set.ptz.iris_plus

To set ptz iris plus parametes

Example: set ptz iris plus parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.iris_plus&chanid=

### 1.10.11 set.ptz.left

To set ptz left parametes

Example: set ptz left parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.left&chanid=&speed=

### 1.10.12 set.ptz.left_down

To set ptz left down parametes

Example: set ptz left down parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.left_down&chanid=&pan=&tilt=

### 1.10.13 set.ptz.left_up

To set ptz left up parametes

Example: set ptz left up parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.left_up&chanid=&pan=&tilt=

### 1.10.14 set.ptz.light_off

To set ptz light off parametes

Example: set ptz light off parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.light_off&chanid=

### 1.10.15 set.ptz.light_on

To set ptz light on parametes

Example: set ptz light on parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.light_on&chanid=

### 1.10.16 set.ptz.preset_clear

To set ptz preset clear parametes

Example: set ptz preset clear parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.preset_clear&chanid=&preno=

### 1.10.17 set.ptz.preset_goto

To set ptz preset goto parametes

Example: set ptz preset goto parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.preset_goto&chanid=&preno=

### 1.10.18 set.ptz.preset_set

To set ptz preset set parametes

Example: set ptz preset set parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.preset_set&chanid=&preno=

### 1.10.19 set.ptz.right

To set ptz right parametes

Example: set ptz right parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.right&chanid=&speed=

### 1.10.20 set.ptz.right_down

To set ptz right down parametes

Example: set ptz right down parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.right_down&chanid=&pan=&tilt=

### 1.10.21 set.ptz.right_up

To set ptz right up parametes

Example: set ptz right up parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.right_up&chanid=&pan=&tilt=

### 1.10.22 set.ptz.serial_port

To set ptz serial port parametes

Example: set ptz serial port parametes

http://192.168.9.129/sdk.cgi/action=set.ptz.serial_port&id=0&baudrate=9600&date_bit=8&stop_bit=1&parity_type=0&protocol=8&address=1&com_type=0&connect_type=0

### 1.10.23 set.ptz.stop

To set ptz stop parametes

Example: set ptz stop parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.stop&chanid=

### 1.10.24 set.ptz.tour_add

To set ptz tour add parametes

Example: set ptz tour add parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.tour_add&chanid=&tourid=&key=&preno=&tour_spd=&tour_time=

### 1.10.25 set.ptz.tour_clear

To set ptz tour clear parametes

Example: set ptz tour clear parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.tour_clear&chanid=&tourid=&key=&preno=&tour_spd=&tour_time=

### 1.10.26 set.ptz.tour_delete

To set ptz tour delete parametes

Example: set ptz tour delete parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.tour_delete&chanid=&tourid=&key=&preno=&tour_spd=&tour_time=

### 1.10.27 set.ptz.tour_down

To set ptz tour down parametes

Example: set ptz tour down parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.tour_down&chanid=&tourid=&key=&preno=&tour_spd=&tour_time=

### 1.10.28 set.ptz.tour_run

To set ptz tour run parametes

Example: set ptz tour run parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.tour_run&chanid=&tourid=&key=&preno=&tour_spd=&tour_time=

### 1.10.29 set.ptz.tour_stop

To set ptz tour stop parametes

Example: set ptz tour stop parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.tour_stop&chanid=&tourid=&key=&preno=&tour_spd=&tour_time=

### 1.10.30 set.ptz.tour_up

To set ptz tour up parametes

Example: set ptz tour up parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.tour_up&chanid=&tourid=&key=&preno=&tour_spd=&tour_time=

### 1.10.31 set.ptz.up

To set ptz up parametes

Example: set ptz up parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.up&chanid=&speed=

### 1.10.32 set.ptz.zoom_in

To set ptz zoom in parametes

Example: set ptz zoom in parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.zoom_in&chanid=&speed=

### 1.10.33 set.ptz.zoom_out

To set ptz zoom out parametes

Example: set ptz zoom out parametes

http://192.168.9.129/sdk.cgi?action=set.ptz.zoom_out&chanid=&speed=

## 1.11 DISK

### 1.11.1 set.disk.format

To set disk format parametes

Example: set disk format parametes

http://192.168.9.129/sdk.cgi?action=set.disk.format&diskid=1

### 1.11.2 get.disk.storage_info

| Parameter | Values | Description |
| --- | --- | --- |
| ret_cnt | integer |  |
| port | integer |  |
| vendor | String max 32 |  |
| total | integer |  |
| user | integer |  |
| free | integer |  |
| state | integer | 0=disk no be format 1=disk is OK, has be formatted 2=disk is not exist 3=disk is formatting 4=disk has error, may be can not be used. 5=loading |
| hddtype | integer | 0=unknow 1=local 2=usb 3=nas 4=raid 5=esata 6=ipsan 7=global spare 8=in raid 9=none |
| property | integer | 0=read only 1=R/W |
| raid_status | integer | 1=disk no exist in system 2=disk not be used for raid array 3=disk used for a raid array 4=disk used for a raid array,but only for spare/resync/recovery function 5=disk exist in a raid array, bug has error, can not be used. 6=disk has be removed |
| raid_level | integer |  |
| raid_task | integer |  |
| raid_diskcnt | integer |  |

Example: get disk storage info parametes

http://192.168.9.129/sdk.cgi?action=get.disk.storage_info

### 1.11.3 set.disk.storage_info

To set disk storage info parametes

Example: set disk storage info parametes

http://192.168.9.129/sdk.cgi?action=set.disk.storage_info&portId=1&recycle_mode=0

### 1.11.4 set.disk.property

| Parameter | Values | Description |
| --- | --- | --- |
| portId | integer |  |
| property | integer | 0=read-only,1=r/w |

To set disk storage property parametes

Example: set disk storage property parametes

http://192.168.9.129/sdk.cgi?action=set.disk.property&portId=1&property=1

### 1.11.5 set.disk.delete

| Parameter | Values | Description |
| --- | --- | --- |
| portId | integer |  |
| type | integer |  |

To set disk storage delete parametes

Example: set disk storage delete parametes

http://192.168.9.129/sdk.cgi?action=set.disk.delete&portId=2&type=3

### 1.11.6 set.disk.recycle_mode

| Parameter | Values | Description |
| --- | --- | --- |
| mode | integer | Recycle mode 0=disable 1=enable |

To set disk storage recycle mode parametes

Example: set disk storage recycle mode parametes

http://192.168.9.129/sdk.cgi?action=set.disk.recycle_mode&mode=0

### 1.11.7 get.disk.recycle_mode

To get disk storage recycle mode parametes

Example: set disk storage recycle mode parametes

http://192.168.9.129/sdk.cgi?action=get.disk.recycle_mode

## 1.12 QT

### 1.11.1 set.disk.format

To control qt

Url:

http://ip/sdk.cgi?action=set.qtctrl.cmd&cmd=XXXXX

Example:

### 1.11.3 set.disk.storage_info

To  LiveView

http://192.168.9.150/sdk.cgi?action=set.qtctrl.cmd&cmd=Page_LiveView

### 1.11.2 get.disk.storage_info

| Parameter | Values | Description |
| --- | --- | --- |
| result | String max 20 |  |
| curPage | String max 20 |  |
| live_view_wnd_num | integer |  |
| play_back_wnd_num | integer |  |
| live_view_select_wnd | integer |  |
| play_back_select_wnd | integer |  |
| live_view_fullScreenWnd | integer |  |
| play_back_fullScreenWnd | integer |  |
| playbackspeed | String max 10 |  |
| playbackmode | String max 20 |  |
| playbackdate | String max 20 |  |
| playbacktime | String max 20 |  |

### 1.11.2 get.disk.storage_info

| Cmd | Action | Description |
| --- | --- | --- |
| Page_XXXX | Go to other page | Page_LiveView, Page_PlayBack |
| FullScreen | FullScreen |  |
| PlayBack_NextHour | Next hour |  |
| PlayBack_LastHour | Last hour |  |
| PlayBack_SpeedUp | Fast forward |  |
| PlayBack_Rewind | Rewind |  |
| PlayBack_LastDay | Last day |  |
| PlayBack_NextDay | Next day |  |
| PlayBack_LastMonth | Last month |  |
| PlayBack_NextMonth | Next month |  |
| PlayBack_Play | Play |  |
| PlayBack_Pause | Pause |  |
| PlayBack_Stop | Stop |  |
| Wnd_X | Split screen | Wnd_4, Wnd_9, Wnd16 |
| SelChannel_XX | Select channel | SelChannel_0, SelChannel_12 (0-15) |
| SelWnd_CENTER | Select window |  |
| SelWnd_UP | Select window |  |
| SelWnd_DOWN | Select window |  |
| SelWnd_LEFT | Select window |  |
| SelWnd_RIGHT | Select window |  |
| SelWnd_XX | Select the wnd | SelWnd_9 |
| Audio | Open or close audio |  |
| SingleChannel_XX | Full the certain channel |  |
| ChangeFocus_Next | Foucs the next item |  |
| Menu | Call the menu |  |
| Esc | Return live view page |  |
| Seq_XX | Open or close sequence | Seq_Main Seq_Sub Seq_Current |